# Maxi Studio Isis Signal Processing Synoptic

## SUMMARY OF ALL THE API AVAILABLE

IsisBankLoad  :

Load a Sound Bank in memory. The index of the new Sound Bank in memory is return as parameter.

IsisBankUnLoad :

Removes the sound bank by Index from board memory

IsisBankGetCaps :

Fills the lpBankCaps structure with the information describing Sound Banks in memory

IsisBankSetPriority :

Change the priority of the sound bank by Index

IsisHwResetDream :

Reset the DSP in Hardware

Option :	combination of the following flags

FIHR_DONT_LOAD_FIRMWARE don't Load default firmware

FIHR_DONT_LOAD_SOUNDBANK don't Load default sound bank

FIHR_DEFAULT default: Load default firmware and sound bank

IsisSetInputDevice :

Select the Input device for the ISIS 7/8 entry

InputDevice:	ISIS_SET_INPUT_CODEC_OUT :			Maestro Main Out

ISIS_SET_INPUT_RACK_ANALOG_78_IN :		Rack Analog In 7/8

ISIS_SET_INPUT_RACK_SPDIF_IN :			SPDIF IN

IsisSetClockFrequency :

Set the Clock Frequency for the DSP

BE CAREFUL Do not Switch to SPDIF IN Clock if there is not Valid SPDIF Clock

So use IsisGetSpdifFrequency before switching to SPDIF IN Clock

ISIS_SET_FREQ_32K_CLOCK :		32 Khz

ISIS_SET_FREQ_44K_CLOCK :		44.1 Khz

ISIS_SET_FREQ_48K_CLOCK :		48 Khz

ISIS_SET_FREQ_SPDIF_IN_SLAVE :		SPDIF IN

IsisGetSpdifFrequency :

Get the Frequency of the SPDIF entry

Frequency :

ISIS_GET_SPDIF_FREQ_OUTOFRANGE :	Out of Range (no Valid SPDIF Entry)

ISIS_GET_SPDIF_FREQ_48_4PCT :		48 Khz ± 4%

ISIS_GET_SPDIF_FREQ_44_10_4PCT :	44.1 Khz ± 4%

ISIS_GET_SPDIF_FREQ_32_4PCT :		32 Khz ± 4%

ISIS_GET_SPDIF_FREQ_48_400PPM :		48 Khz ± 400 ppm

ISIS_GET_SPDIF_FREQ_44_10_400PPM :	44.1 Khz ± 400 ppm

ISIS_GET_SPDIF_FREQ_44_05_400PPM :	44.056 Khz ± 400 ppm

ISIS_GET_SPDIF_FREQ_32_400PPM :		32 Khz ± 400 ppm

IsisSwitchConfig :

Switch to MUSIC or MULTIMEDIA Configuration

ISC_TO_MUSIC :	Music Configuration

ISC_TO_MMEDIA :	Multimedia	Configuration

IsisGetInputDevice :

Get the Selected Input device for the ISIS 7/8 entry

Input Device :ISIS_INPUT_CODEC_OUT :	Maestro Main Out

ISIS_INPUT_RACK_ANALOG_78_IN :		Rack Analog In 7/8

ISIS_INPUT_RACK_SPDIF_IN :			SPDIF IN

IsisGetClockFrequency :

Get the Clock Frequency of the DSP

Clock :	ISIS_FREQ_32K_CLOCK : 32 Khz

ISIS_FREQ_44K_CLOCK : 44.1 Khz

ISIS_FREQ_48K_CLOCK : 48 Khz