ID_RING Version 1.0

FORWARD

-------

I wanted to share my voice line with my computer which was running remote

communication software (i.e. PC Anywhere or Close-Up).  My choices were

to either set up my modem to answer after many rings (ATS0=10) or to

order "Identa-Ring" service from my local phone co. and then install a

$90 diverter box.  I decided there must be another option....

DESCRIPTION

-----------

ID_RING is a resident utility (uses 4K) that monitors the modem's ring

indicator line and issues an ATA command through the serial port if the

correct ring pattern is detected.  It writes directly to the hardware

bypassing any re-directed interrupts that your comm software may have set

up.  You must order "Identa-ring" distinctive ring service from your

local phone company.

WHAT IS DISTINCTIVE RINGING

---------------------------

When you set up "Identa-ring" service you will be given a second (or even a

third) phone number that rings through your existing phone line.  You will

not have two or three phone lines, just more that one phone number for the

one line you have.  When someone calls the first number the phone will

ring normally.  When someone calls the second number there will be two

shorter rings instead of one normal ring.  If you set up a third number

there will be three rings. ID_RING listens for the special ring pattern

and tells the modem when to answer.

USAGE

-----

ID_RING COMn [PATn] [QUIET=n]

COMn is required.

Enter COM1, COM2, COM3, or COM4. Use whatever

port your comm software is running on.

PATn is an optional parameter.

ID_RING answers on the default "ring-ring" pattern.  If you want to

answer on the "ring" pattern use PAT1 or if you want to answer on the

"ring-ring-ring" pattern use PAT3.

QUIET=n is an optional parameter.

ID_RING looks for 36/18ths of a second (two seconds) of silence between

rings.  If ID_RING is not working properly you can adjust the quiet time.

n=18ths of a second.

NOTES

-----

Load ID_RING just before loading your comm software.

Unload ID_RING after you unload your comm software or the modem may

continue to answer the phone.

You should change your comm software modem init string so that S0=0.

SUPPORT

-------

Contact the author:

John Hutchey

RR #1  Box 752

Lake Ariel, PA  18436

Email:

JHutchey@AOL.com or 70247,704@CompuServe.com