Installation notes for Windows95.

Installation for Windows95 is in two parts:

(I). Installation of the drivers (Perform all steps A thru D)

Step A - Remove existing audio device drivers

Step B - Add audio device drivers

Step C - Add Gameport\Joystick device driver

Step D - Add Dos support under Windows95.

(II). Installation of the applications

Step A - Add Windows Applications (mixer, talking clock, etc.)

I - INSTALLATION OF THE DRIVERS

Start Windows95

Step A - Remove any existing audio device drivers

(1). Open "Control Panel"

(use Start->Settings->Control Panel)

(2). Open "System" in Control Panel

(3). Select "Device Manager" top tab

(4). Look for the entry "Sound, video and game controllers"

If the entry does not exist:

- Select "Cancel"

- Go to Step B

If the entry does exist, then continue with (5).

(5). Select the plus sign to the left of the entry

(6). Look for any sound card entry (such as SoundBlaster or Audiodrive)

If the entry does not exist:

- Select "Cancel"

- Go to Step B

If the entry does exist, then continue with (7).

(7). Highlight the sound card entry (such as SoundBlaster or Audiodrive)

- Select "Remove"

- Select "Yes" to confirm device removal

- Select "Close", continue with Step B

Step B - Add audio device drivers

(1). Open "Control Panel"

(2). Open "Add New Hardware" in Control Panel

(3). Select "Next>"

(4). Do You want windows to search for your hardware?

- Select "No"

(5). Select "Next>"

(6). Highlight "Sound,Video and game controllers"

(7). Select "Next>"

(8). Select "Have Disk"

(9). Put the Sound Card Disk-1 into your drive A:

(10). Select "Browse"

(11). Highlight "WIN95" folder and select "OK"

(12). "OEMSETUP.INF" will automatically be highlighted, select "OK"

(13). The path will show A:\WIN95, select "OK"

(14). The Sound Card will be highlighted, select "OK"

(15). Select "Next>"

(16). Select "Finish"

(17). Do you want windows to shut down your computer now?

- Select "No", continue with Step C

Step C - Add Gameport/Joystick device driver

(1). Open "System" in Control Panel

(2). Select "Device Manager" from the top tabs

(3). Look for the entry "Sound, video and game controllers"

(4). Select the plus sign to the left of the entry

(5). Look for the entry "Gameport/Joystick"

If the entry exists:

- Select "Cancel"

- Go to II - INSTALLATION OF APPLICATIONS

If the entry does not exist, then continue with (6).

(6). Select "Cancel"

(7). Open "Control Panel"

(8). Open "Add New Hardware" in Control Panel

(9). Select "Next>"

(10). Do You want windows to search for your hardware?

- Select "No"

(11). Select "Next>"

(12). Highlight "Sound,Video and game controllers"

(13). Select "Next>"

(14). Highlight "Microsoft"

(15). Highlight "Gameport/Joystick"

(16). Select "Next>"

(17). Select "Next>"

(18). Select "Finish"

(19). Do you want windows to shut down your computer now?

- Select "No", continue with step D

Step D - Add DOS support under Windows95

PUT CHIPCHAT DISK-1 INTO YOUR FLOPPY DRIVE

(1). Open "MS-DOS Prompt"

(2). Put the sound card's DISK-1 into your floppy drive

(3). Type "A:INSTALL"

(substitute proper drive for "A:" if necessary)

(4). Be sure to read the document files at the end of installation

(5). Exit from the MS-DOS Prompt

II - INSTALLATION OF THE APPLICATIONS

Step A - Add Windows Applications (mixer, talking clock, etc.)

PUT CHIPCHAT DISK-2 INTO YOUR FLOPPY DRIVE

(1). Select "Start" then "Run"

(2). Use Browse to change to your floppy drive

(3). Select "WINAPPS.EXE" to run

SHUT-DOWN AND RESTART WINDOWS95

After you have loaded the appropriate drivers and re-started Windows95

use the Windows applications loaded in step II to play digitized audio,

play MIDI songs, and adjust the volume with the Mixer.

If you have the WaveTable option, you can change from the FM synthesizer

to the WaveTable synthesizer using the Control Panel.

(1). Open the "Control Panel"

(2). Open "MultiMedia" in the Control Panel

(3). Select "MIDI" top tab

(4). To choose WaveTable:

Highlight "MIDI - WaveTable Synthesis and External Port"

To choose FM:

Highlight "MIDI - FM Synthesis"

(5). Select "OK"

You will have to stop and re-start the song playing for the change

to take effect.