Slick is a Copy(c)right 1985,1994 by Gary Raymond, New Orleans, La.

Slick Modem Terminal Program V6.5a

Tech Help 504-288-6550

UP AND DOWN LOADING:

Slick is simple, very powerful, and easy to use! It has everything

you basically need to enjoy Telecommunication, including the most

popular protocols like Zmodem (the best to use!) With Zmodem,

you need only instruct the host to begin the Zmodem transfer,

SLICK does the rest! All other protocols require hitting the PgDn

key (for down loading) and selecting the correct protocol. Today,

99% of all BBS's are providing Zmodem file transfers because of

the superior performance it has over X and Ymodem. Zmodem has

become the de facto standard on almost all Bulletin Boards. But,

if X and Ymodem are still needed, SLICK has them as well.

Before you start trying to transfer files its a good idea to first

establish a directory to receive down loaded files as well as a

directory to place files that you intend to send or upload. This

can be done through the Default Menu reached via Alt-Z. For ease

of creating DOS directories, two facilities are provided. One is

Create DIR and the other is Kill DIR. After deciding if you can

use directories that already exist or that you must create new

directories, you will then need to define those directories so

SLICK will know what to use. Alt-D and Alt-U in the Default's Menu

will allow you to define the directory locations.

To upload (or send), FIRST tell the remote you are about to send

using a particular protocol. Next hit the PgDn key and select the

same protocol on your end from the menu (see below).

��������������������������������������������ͻ

�    ������ĴProtocol Upload Menu��������    �

�                                            �

�  [X]modem  [Y]modem Batch    [G]Ymodem G   �

�  [S]ealink [A]scii [Z]modem  [K]1K Xmodem  �

�                                            �

�   Select Protocol -[Esc or Cr Exits]->     �

��������������������������������������������ͼ

As soon as you select the matching protocol, the following window

will appear allowing you to cherry pic the file or files you wish

to send. If you are using Xmodem, you are restricted to one file

transfer at a time. As you select a file or files to send, you

will note they become marked, indicating they have been included

in the batch to send. The rest is automatic. When the transfer is

complete, you will be returned to the BBS for additional activity.

���������������������������������������������������������������ͻ

� Use Arrows, PgUp, PgDn to Navigate. Select with the SPACEBAR. �

� Hit <Escape> to ABORT!  Use <Enter> when selections are over. �

���������������������������������������������������������������ͼ

�͵ Upload Dir = Default ��������������������������������������ͻ

�  SLICK          DO.BAT         SCREEN.CUT     SCREEN.CU1      �

�  SCREEN.CU2     SCREEN.CU3                                    �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

���������������������������������������������������������������ͼ

To download, first tell the remote what file or files you want as

well as the protocol you intend to use. Remember, with Xmodem, you

are limited to receiving one file at a time. Next, hit the PgDn key

and the menu shown below will appear. Select the same protocol you

indicated to the remote. NOTE: For Xmodem, you must also name the

file to be received on your end. You will automatically be prompted

to do so. The name *DOWNFILE.NAM will appear by default in the

window. Simply over type or delete. This is one of the reasons you

should allow Xmodem to go to protocol heaven and use Ymodem B or

preferably, Zmodem!

��������������������������������������������ͻ

�   ������ĴProtocol Download Menu��������   �

�                                            �

�  [X]csum,1kcrc [Y]modem Batch [G]Ymodem G  �

�  [S]ealink     [A]scii Text   [Z]modem!    �

�                                            �

�   Select Protocol -[Esc or Cr Exits]->     �

*�    Enter File Name ----> DOWNFILE.NAM      �

��������������������������������������������ͼ

Some Protocol History and Facts.

Xmodem

======

Ward Christensen originated the Xmodem protocol many years ago as

a simple solution to transferring files between two computer

systems. Over the years, the Xmodem protocol developed into the

first de-facto standard for file transfers between microcomputers.

The very first Xmodem used a very simple checksum (CSUM) method of

insuring the integrity of each block transmitted and received.

Today, many variations of Xmodem have appeared which address a

number of the limitations and performance problems of the original

Xmodem protocol. Modern Xmodem has both Checksum and the more

accurate cyclic redundant check (CRC) for insuring file data

accuracy. The newer version of Xmodem with both CSUM/CRC is now

the standard in use with most all remote bulletin board systems

and mini-computer host systems.

Unfortunately, these are single-file-at-a-time transfer methods.

Files are transmitted in 128 character blocks along with the block

number and checksum or cyclic redundancy check (CRC) information.

This allows the receiving system to ensure that the transmission

has occurred correctly to a very high degree of probability. The

CRC error-checking method is superior to the Checksum method, so

it is always preferable to use. It should be automatic on most

remote systems (BBS'es) but if a choice is required between XModem

csum and Xmodem crc, pick Xmodem crc.

If you are resistant to using more accurate and efficient

protocols, you should be aware that mean ordinary Xmodem  imposes

very severe timing constraints on inter-character and inter-block

transmission delays. Some systems which implement Xmodem require

that these timing constraints be relaxed. In addition, because it

transmits in 128 byte blocks, it is likely to append a bunch of

nulls to the end of the received file in order to pad the last

packet. This will result in receiving a file that is longer that

the one transmitted. Although harmless, it takes up more space on

your drive and more time to transmit.

Xmodem 1K

=========

Something a little better. Xmodem 1K is an Xmodem variant which

provides for long 1024 character blocks -- more efficient for

higher baud rates like 2400, 4800, and 9600 baud. Like Xmodem and

Xmodem CRC, Xmodem 1K only provides for the transfer of one file

at a time. Because of some confusion over terminology, Xmodem 1K

is sometimes referred to as plain Ymodem. However, true Ymodem

(Ymodem/B) is actually a batch protocol that allows several files

to be transmitted in sequence. See the discussion of Ymodem below

for details.

Something better yet. A variant of Xmodem 1K, called Xmodem G is

available. Its purpose is to take advantage of hardware error

correction. Therefore it will only work correctly between two

error-correcting modems. This protocol operates just like regular

Xmodem but allows for a continuous flow of blocks to be

transmitted by the sending computer. XON/XOFF flow control and/or

CTS flow control are used to synchronize the two computer systems.

Again, this protocol relies on the two error-correcting modems to

provide the necessary block re transmissions in case of error.

However, if an error occurs between the modem and the

microcomputer (which does happen), then Xmodem 1K G has to give

up, since there is no provision for resending bad blocks under

program control.

Ymodem B

========

Something almost perfect. Ymodem Batch is another Xmodem variant

which provides for sending the file name, creation time and date,

and exact file size. This information is contained in a special

"block 0" which is prepended to the actual data of the file

itself. Ymodem allows for the transfer of several files at once.

In general, the actual data part of a file is sent using Xmodem

1K-style long blocks with CRC error checking.

Zmodem

======

Something PERFECT! In general, the Zmodem protocol is a better

choice for a protocol to use with or without error-correcting

modems, since it provides the streaming speed of Xmodem 1K G with

error recovery and re transmission of bad blocks under program

control. An increasing number of bulletin board systems are now

providing the Zmodem protocol. Zmodem is a full-duplex protocol

that provides excellent throughput and exceptional error-recovery

even on poor connections. Zmodem also allows you to restart a file

transfer that aborted because of a dropped carrier. For example,

suppose that you were transferring a 380K file, and after 320K the

carrier dropped. With other file transfer protocols, you would

need to restart the transfer from the beginning. Ugh! With Zmodem

it is possible to reconnect to the remote system and pick up the

transfer where it left off, so that only the remaining 60K of the

file needs to be sent, rather than the entire 380K all over again.

SeaLink

=======

If you need it, its here. Its similar to Ymodem B in operation and

not much of an improvement on anything other than being a nuisance

protocol. It was developed by System Enhancements Associates to

avoid copy right problem with some other proprietary software

already on the market. It's around, but apparently not too

popular.

FINAL NOTES:

Slick's Protocols will Abort after 10 encounters with line spikes

(resulting in crc errors that require retransmission) or after 10

attempts to handshake with the protocol on the other end with no

positive results. With Zrecover, you can simply call back when the

lines improve, and finish where you left off.

----------------------------------[EOF]-----------------------------------