AVPLAN

Version 3.0

(C) Copyright  1987, 1989

Written by

Dale C. Metz

INDEX

Disclaimer ........................................... 1

Introduction ......................................... 2

Requirements ......................................... 2

Installation ......................................... 3

MAIN MENU

Main Menu (content) .................................. 4

Creating a flight log ................................ 4 - 5

Calculating wind corrections and ground speeds........ 5

Performing weather calculations ...................... 6

Performing weight and balance ........................ 6

Creating a flight plan form .......................... 6

Calculating a cost analysis .......................... 7

DISK OPERATIONS

Disk operations (content) ............................ 7

View waypoints in database ........................... 7

Retrieving and deleting flight logs .................. 7

Saving a flight log .................................. 8

Sorting a waypoint datafile .......................... 8

Defining default entries for the flight plan form .... 8

Program setup - drives, directories, colors, mouse ... 8

Exit to DOS and return ............................... 9

PRINT OPERATIONS

Print menu (content) ................................. 9

AVPLAN DATABASE

Avplan program and data files listing ................ 9

Database description and discussion .................. 9 - 10

Known program bugs ................................... 10

Warranty disclaimer .................................. 11

PROGRAM CHANGES

Version 2.0 .......................................... 12

Version 2.2 .......................................... 12 - 13

Version 3.0 .......................................... 13

Avplan order form .................................... 14 - 15

i

AVPLAN Version 3.0

___________________________________________________________________________

AVPLAN Version 3.0       06/01/89  (C) Copyright  D.Metz

*****************************************************************

THIS PROGRAM IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND.

THE ENTIRE RISK AS TO THE RESULTS AND PERFORMANCE OF THE PROGRAM

IS ASSUMED BY YOU.  THE AUTHOR DOES NOT WARRANT, GUARANTEE, OR

MAKE ANY REPRESENTATIONS REGARDING THE USE OF, OR THE RESULTS OF

THE USE OF, THE PROGRAM IN TERMS OF CORRECTNESS, ACCURACY,

RELIABILITY, CURRENTNESS, OR OTHERWISE; AND YOU RELY ON THE

PROGRAM AND THE RESULTS SOLELY AT YOUR OWN RISK.  THIS PROGRAM IS

AN AID TO FLIGHT PLANNING AND IS NOT INTENDED TO BE USED AS THE

SOLE RESOURCE FOR FLIGHT PLANNING.

*****************************************************************

1

AVPLAN Version 3.0

___________________________________________________________________________

INTRODUCTION

AVPLAN was written to assist the pilot in preflight planning.

Much care has been taken to assure the accuracy of the program

but the possibility of errors always exists.  Because of the

importance of this possibility, it is essential that this program

be used only as an aid during your preflight planning.  All

program results should be verified with other flight planning

devices and aids.  The data listed in the database is subject to

change.  Database updates may eventually be available for

registered users but this will depend on the interest and support

for the program.

This program was designed to provide some basic information

for the pilot in his flight planning phase.  In talking with

general aviation pilots, I have concluded that many of these

pilots place a limited role on computer flight planners.  I too

question the importance of flight planners incorporating every

facet of aircraft performance when calculating flight plan

output.  The reality is that flight planners can only provide

predictions.  The pilot using this data, must monitor all aspects

of the flight and vary those predictions as required.  Variables

such as winds and ATC routing can alter the flight plan data

drastically.  With this in mind, I have tried to create a program

that is easy to use and requires a minimal amount of input and

data to provide some basic flight plan predictions.  I hope that

you find the program effective in meeting this objective.

HARDWARE REQUIRED

o  IBM or compatible

o  DOS v2.1 or higher

o  dual floppy or a hard drive system

o  512K or higher

o  optionally, a mouse

2

AVPLAN Version 3.0

___________________________________________________________________________

INSTALLATION

An installation batch file has been included for those

persons installing AVPLAN in a subdirectory on a hard drive or on

a high capacity floppy disk. - (720K or greater).  The batch file

INSTALL.BAT, will execute the installation program for AVPLAN. If

you are using a hard disk, you should run INSTALL.BAT from DOS.

You will need to specify a single drive and directory for your

program and data files.  The directory will be created and

verified and all of the files copied to that directory.   Insert

the program and data disk as directed during installation.

eg.  from DOS type INSTALL.BAT and press the <ENTER> key to begin

program installation.

If your files are to be copied to a single floppy you should

exit the INSTALL routine when directed by holding the control key

down and pressing "C".  Then use the DOS copy command to copy all

of the files from your AVPLAN disk onto a single floppy.  If you

are running AVPLAN from a single floppy drive then you should not

specify a drive and directory for the program unless you will

always use that same drive and directory.

If you inadvertently specify a drive and directory that is

inaccurate or unnecessary you may create a new AVPLAN.PRO file.

Delete the old file in DOS using the command DEL AVPLAN.PRO.

Execute the program AVINSTAL.EXE from DOS and a new AVPLAN.PRO

file will be created.

To copy your AVPLAN files from disk A: to B: - (AVPLAN in A:

drive)

COPY *.* B:

Do this for both the program and data disk.

If your are operating from dual 360K drives, you will need to

specify the floppy drive where the program will find the waypoint

or airport files.  These files have the extension "FIL" and will

be found on the data disk.   The drive letter can be assigned

using the option to select a drive and directory when executing

the installation batch file, or AVINSTAL.EXE, or select - <7>

Disk Operations from AVPLAN's main menu and then - <6> Program

Setup and Colors.

3

AVPLAN Version 3.0

___________________________________________________________________________

I.  USING AVPLAN

The program provides a main menu for selection of program

applications.  Of these choices, some will provide a sub menu

allowing more specific choices.  The main menu choices are as

follows:

[1]  Flight Log

[2]  Wind Corrections

[3]  Weather Calculations

[4]  Weight and Balance

[5]  Flight Plan

[6]  Cost Analysis

[7]  Disk Operations

[8]  Print Operations

[X]  Exit to DOS

F1 - Provides help screens for each menu.  <ESC> - The

escape      key allows you to exit most screens and menus.

Flight Planning usually begins by loading or creating a

flight      log

[1]  FLIGHT LOG

This selection will allow you to enter your waypoint

identifiers for retrieval from the database.  The waypoints are

identified in the database using standard FAA designations,

usually three characters.  For example Orlando Executive Airport

is listed as `ORL'.  If you are specifying the VOR as your

waypoint, add `V' to the identifier, in this example `VORL'. See

the section describing the database for additional information.

If you are not sure of the designation for your waypoint you

may enter <L>ist at the prompt.  This will allow you to list all

of the available waypoints by state, using a two letter data-file

identifier.  You then return to the flight log screen to complete

your log.

If the desired waypoint is not in the database, you may be

enter the waypoint.  A name, the latitude and longitude

coordinates, magnetic variation and the frequency are required to

enter a waypoint.  You will be prompted then to save the

waypoint, continue without a save, or to abandon the operation.

You will then be returned to the screen for flight log creation.

Ten waypoints are allowed in any one flight log.  When you have

completed your waypoint entries  you may press the <ENTER> key

only or <D>one at the prompt.  A screen will appear requesting

your true airspeed, gallon usage per hour and time to allot for

climb-out.  The log will then be computed and displayed.

4

AVPLAN Version 3.0

___________________________________________________________________________

====================[ Please Note ]=====================

The log will be based on your True Airspeed entry and may be

inaccurate if your groundspeed varies greatly.  You can figure

groundspeed corrections based on winds aloft estimates using that

selection in the main menu. You will be prompted to recalculate

the flight log based on your groundspeed selection from the

groundspeed and wind correction display.

[2]  WIND CORRECTIONS

To figure wind correction after you have created a flight

log or loaded a previous log from your program/data disk,

select: WIND CORRECTIONS from the main menu.  It is not necessary

to have a flight log in memory to use this option.  You can

directly enter the headings and magnetic variations.  If you

choose to do this, the flight log in memory will be lost, so save

it if it will be used again.

You may enter the wind velocities and direction for each

altitude between the surface and 12,000  feet at 3,000 feet

increments.  Enter the true airspeed for your aircraft.  The

program will provide the groundspeeds and headings for each

course and altitude, correcting speed, and heading for the wind

conditions specified for your log.  Also provided is the distance

of each leg and estimated time in route for each altitude.  You

may select <R>ecalculate Flight Log to enter the groundspeed you

desire based on the information provided.  You may return to the

Wind Correction and Groundspeed screen by selecting that choice

from the Print Operations Menu.

=====================[ Please Note ]======================

Wind corrections and groundspeed calculations are done on the

entire log and do not allow for separate wind entries on a leg by

leg basis.  This should be adequate for most general aviation

applications and makes for quick flight planning.  If winds aloft

change significantly from departure to destination it will be

necessary to create more than one log and combine the totals.

This allows wind corrections based on winds-aloft-estimates for

specific legs.

[3]  WEATHER CALCULATIONS

A sub menu is provided allowing for simple weather

conversions and utilities.  The facilities in this area may be

expanded in subsequent versions.

5

AVPLAN Version 3.0

___________________________________________________________________________

[4]  WEIGHT AND BALANCE

This selection calls a routine that computes the total weight

and center of gravity for your aircraft.  The program design

allows seven stations for passenger and baggage loading and two

stations for fuel.  The program automatically calculates fuel

weight based on the fuel quantity in gallons.  The routine

requires the user to enter the weight and arm for each station

involved. This information can be saved for each aircraft and

does not need to be reentered.  The file for aircraft data will

hold information on forty aircraft.  It is IMPERATIVE that the

data entered in this program be accurate from the aircraft weight

and balance charts and data sheet.  The results should be

compared with loading charts to insure they are within acceptable

limits.

REMEMBER:  Weight x Arm = Moment

Some aircraft manuals or weight and balance sheets may not

specify the ARM figure for the aircraft or each station.  The ARM

can be calculated by dividing the moment by weight.

[5]  FLIGHT PLAN

This option provides a screen that allows you to list a

flight plan using stored values.  If you do not use these

defaults, then you will have to enter manually all the

information for the plan.  This screen display resembles the FAA

flight plan form 7233-1.  You should enter the information for

each category as you would in filling out the FAA Flight Plan

form.  The stored values are loaded from the file `PLAN.DAT'

which must be on your data disk and can be created by selecting:

ENTER DEFAULTS FOR FLIGHT PLAN from the Disk Operations Sub Menu.

If you have previously created a flight log or loaded one from

disk, some of the data will be automatically inserted into the

plan.  If you wish to change this data type over it when prompted

for that category.  This form can be printed and used for

reference.  I have not used this form in an official capacity for

filing a flight plan and therefore cannot anticipate the response

from various FSS offices.  It is my hope that they will accept

the form but I cannot assure they will.

6

AVPLAN Version 3.0

___________________________________________________________________________

[6]  COST ANALYSIS

To execute this routine you must have a flight log in memory

from previous execution or load one from your program/data disk.

The steps for running a cost analysis are simple to follow and

will provide you with a rough cost analysis based on your true

airspeed.  This does not take into account your ground time. The

analysis will probably reflect your cost most accurately if you

use a tach time figure for aircraft cost, especially if renting.

As an aircraft owner you may specify fuel cost, engine reserve

time, and miscellaneous cost of operation towards calculating the

flight cost for a flight log.

[7]  DISK OPERATIONS

Selecting this option from the Main Menu will provide the

following Sub Menu:

[1]  View Waypoints in Database

[2]  Retrieve/Delete a Flight Log

[3]  Save Flight Log to Disk

[4]  Sort Waypoints in Database

[5]  Enter Defaults for Flight Plan

[6]  Program Setup and Colors

[7]  Exit to DOS and Return

[6]  Return to Main Menu

<ESC> - escape will return to main menu

VIEW WAYPOINTS IN DATABASE  -  Prompts you to enter the two

letter file or state identifier for the data file you would like

to view. The state abbreviations used in the creation of the

database are those listed by the U.S. Post Office.  For example,

Florida would be abbreviated FL and is in the database as FL.FIL.

The <Pg Up> and <Pg Down> keys allow you to browse up and down

the waypoint file names to locate an airport or waypoint.  You

can return to the screen for creating flight logs by pressing

<RETURN> or press <ESC> for the Main Menu.

RETRIEVE/DELETE A FLIGHT LOG   -  Will list all available flight

logs from your program/data disk.  You then load a file into the

program by typing the filename at the prompt exactly as it

appears.  To delete a log simply type `DELETE' `filename' where

`filename' is the name of your log.

7

AVPLAN Version 3.0

___________________________________________________________________________

SAVE FLIGHT TO DISK  -  Stores your flight log to the disk naming

it with the departure waypoint and the destination waypoint.  A

trip to Tallahassee from Orlando would be named  ORL-TLH .  There

is no file extension.  Note that because a file is limited to

eight characters, two VOR waypoints might appear like this:

VORL-VTL where the H in VTLH has been dropped.

SORT WAYPOINTS IN DATABASE  -  Allows the user to sort the

database according to waypoint name.  The sort is not based on

the waypoint designation!  This routine may be useful if you have

entered additional waypoints from the program.  The data file is

sequential and all additions are appended to the end of that data

file.  This sort operation allows the file to be recreated in

alphabetical order based on the waypoint name for ease of

retrieval when creating a flight log.

ENTER DEFAULTS FOR FLIGHT PLAN  -  This selection will create the

file PLAN.DAT, which contains the data to be automatically loaded

into your flight plan form if you so desire.  DO NOT use commas

when entering information at the prompts in this screen.  To do

so will result in inappropriate assignment of information into

your flight plan.

PROGRAM SETUP AND COLORS -  The user may define program colors or

select monochrome operation.  The colors may be chosen from three

colorsets.  There is also a choice which enables the user to

specify the colors that he prefers for program operation.

An option to specify mouse support in using AVPLAN is also

available.  The mouse support offered in this version is in the

initial stages of development and further refinement will

probably take place.  AVPLAN supports two button mouse control.

The left button is used for menu or activity selection.  The

right button is used to provide a <RETURN> keystroke when

required.

You may select a default disk drive and directory from which to

run AVPLAN.  It is particularly important that this be done if

you use the Exit to DOS facility.  Rejoining AVPLAN after an exit

to DOS and a change in drive or directory would prevent the

program from finding the necessary files for operation.  Defining

these defaults will overcome this problem.  When specifying the

drive and directory from which to run the program you may also

define the disk drive and directory for your data or waypoint

files.  When specifying the drive, enter only the drive letter.

When typing the directory, if one is used, enter a backslash, "\"

and the directory name.  eg; \AVPLAN   Do not include a backslash

at the end of the subdirectory name.

8

AVPLAN Version 3.0

___________________________________________________________________________

EXIT TO DOS AND RETURN -  The user may drop to DOS, perform DOS

operations, and return to the program.  Type EXIT to return to

AVPLAN.  The default drive and directory should be specified in

setup or you must return to the AVPLAN drive and directory before

rejoining the program.

[8]  PRINT OPERATIONS

This selection in the Main Menu allows you to print to screen

each of the data screens as listed below:

[1]  Flight Log

[2]  Flight Plan

[3]  Wind Correction and Groundspeed

[4]  Cost Analysis

[5]  Return to Main Menu  or  <ESC>

You may then type <P>rint at the prompt to print a hardcopy.

**** To do repetitive Flight Log calculations specifying

different groundspeeds, you must recall the Wind

Correction and Groundspeed screen from this menu.

ABOUT THE DATABASE

Avplan30 is being distributed in Archived format.  The

Archive should contain the following files:

-  AVPLAN30.EXE  -  executable program file

-  AVINSTAL.EXE  -  required to create avplan.pro in setup

-  WTBAL10.EXE   -  program for weight and balance

-  AC.DAT        -  weight and balance data file

-  INSTALL.BAT   -  batch file for program installation

-  AVPLAN30.DOC  -  avplan documentation

-  AVPLAN.HLP    -  avplan help file for program access

-  DATAFILE.ZIP  -  archive contains state data files

-  AVPLAN.PRO    -  file contains program setup data

-  PRINTDOC.BAT  -  allows printing AVPLAN.DOC to printer

You must de-archive the file  DATAFILE.ZIP in order to use

the program as this contains the data base.  This file will

produce 55 individual files.  All of the United States are

included plus Canada (CN.FIL), Puerto Rico (PR.FIL) the Bahamas

Islands (BI.FIL) and the Virgin Islands (VI.FIL)

All fifty-one of the state files contain data but the four

listed above are empty.  It is necessary, however for the file to

be present on the data disk if you are to add a waypoint from

within the program.  The files are therefore included on the

disk.

9

AVPLAN Version 3.0

___________________________________________________________________________

The database files are sequential in design.  There is no way

of modifying the data files through Avplan, except by sort and

append operations.  Any deletions or other file maintenance will

need to be accomplished through an ascii file editor or ascii

word processor.  You must review the database for accuracy if you

receive it via a public mode such as over a bulletin board.

THERE IS NO CONTROL OVER HOW PUBLIC FILES MAY BE MODIFIED.

The data file structure is as follows:

WP Designation, WP Name, Lat Degrees, Lat Minutes, Lon Degrees,

Lon Minutes, Magnetic Variation, Waypoint Frequency

eg.  ORL,ORLANDO EXECUTIVE,28,33,81,20,2,118.7

or   VORL,ORLANDO EXEC VOR,28,33,81,20,2,112.2

Latitude and Longitude coordinates in the original database

are accurate to within .5 seconds.  Magnetic Variations are

accurate, in most cases, to within 2 degrees.  Frequencies are

listed in order of priority and availability as follows.

- Tower frequency (controlled airport)

- Unicom frequency (uncontrolled airport)

- Clearance delivery

- None if unavailable - 000.0

- If VOR Waypoint - VOR frequency

You must confirm listed frequencies with current charts,

facilities directories or other official sources for this

information.

=====================[ Please Note ]======================

Avplan version 1.0 was distributed in the Central Florida

area only.  I have discovered a critical bug in that version and

request that any copies of the program be destroyed.  I will

provide you with a copy of version 3.0 on request.

Avplan version 2.0 has a known problem with file appends.

You will notice this in both AVPLAN20.EXE  and FILTYPE.EXE.  A

problem in the compiler used on that version caused the end of

file character to remain at the same location in the file even

after an append operation.  Any attempts to add airports or

waypoints through these programs were unsuccessful.  The data

file would grow in size but the new information could not be

accessed by the program.  This problem has been corrected in

version 2.2.

10

AVPLAN Version 3.0

___________________________________________________________________________

AVPLAN Version 3.0       06/01/89  (C) Copyright  D.Metz

*****************************************************************

THIS PROGRAM IS PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND.

THE ENTIRE RISK AS TO THE RESULTS AND PERFORMANCE OF THE PROGRAM

IS ASSUMED BY YOU.  THE AUTHOR DOES NOT WARRANT, GUARANTEE, OR

MAKE ANY REPRESENTATIONS REGARDING THE USE OF, OR THE RESULTS OF

THE USE OF, THE PROGRAM IN TERMS OF CORRECTNESS, ACCURACY,

RELIABILITY, CURRENTNESS, OR OTHERWISE; AND YOU RELY ON THE

PROGRAM AND THE RESULTS SOLELY AT YOUR OWN RISK.  THIS PROGRAM IS

AN AID TO FLIGHT PLANNING AND IS NOT INTENDED TO BE USED AS THE

SOLE RESOURCE FOR FLIGHT PLANNING.

*****************************************************************

The author grants limited permission to copy and distribute this

program provided that the following conditions are met:

1:  THE PROGRAM OR ITS DOCUMENTATION MAY NOT BE CHANGED OR

ALTERED

2:  THE PROGRAM MUST BE DISTRIBUTED WITH THE PROGRAM

DOCUMENTATION

3:  THE PROGRAM MUST BE DISTRIBUTED ON A NONPROFIT BASIS

4:  THE DATA FILES MUST BE DISTRIBUTED WITH THE PROGRAM IN AN

UNALTERED CONDITION

5:  THE PROGRAM AND OTHER FILES MUST BE CIRCULATED IN THE

UNALTERED ARCHIVED FILE CALLED AVPLAN30.ZIP

*****************************************************************

11

AVPLAN Version 3.0

___________________________________________________________________________

CHANGES

VERSION 2.0

-  Compiled September 1987

-  Major aesthetic changes to the program

-  Corrected bug in Version 1.0 that caused errors

in heading calculations

-  Provided submenus for disk and file functions,

weather utilities, weight and balance, and print

functions  (weight and balance not supported)

-  Reorganized the sequential file structure in the

data files

-  Added routine to print Flight Plan form similar to

FAA form 7233-1

-  Included a routine to establish defaults for the

Flight Plan

-  Provided options for sorting waypoint datafiles in

alphabetical sequence on first letter only

-  Added a help routine for context sensitive help from

within the menus

-  Distributed with FILTYP20.EXE for intensive database

expansion

-  Database includes some entries for almost all of the

fifty state files

VERSION 2.2

-  Compiled January 1989

-  Changes in program aesthetics that include growing,

shaded windows

-  Added the file AVINSTAL.EXE for configuring program

colors, default directories and mouse control

-  Added limited mouse support

-  Restructured the program code to a structured basic

12

AVPLAN Version 3.0

___________________________________________________________________________

CHANGES (Continued)

-  Excluded FILTYPxx.EXE  starting with this version

-  Resolved a previous problem with file appends to the

data files

-  Program Linked with ADVBAS.LIB routines

-  Eliminated the need to enter waypoints in capitals

-  Added capability to Drop to DOS and return

-  Corrected integer problems in the weather routines

VERSION 3.0

-  Compiled September 1989

-  Corrected a bug in version 2.2 that resulted in an

infinite loop when using a mouse and adding a

waypoint to the database

-  Added a weight and balance program

-  Database listing includes coordinates, variation, and

frequency

-  Added forward and reverse scrolling to database listing

and help screens

-  Provides for calculation and display of a return trip

-  Allows total of ten waypoints

-  Additional minor aesthetic changes

-  Allows specification of database drive and directory

-  Structure of AVPLAN.PRO changed

-  User specific colors available in setup

-  Improved routines for data entry

-  Improved datafile sort routine

-  Added over 1000 VOR waypoints to the database

-  Changed the file structure of flight logs saved to disk

(past version flight log files will not work with version 3.0)

13

AVPLAN Version 3.0

___________________________________________________________________________

If you are interested in receiving information about new

versions of AVPLAN send a SASE to this address:

DCM ENTERPRISES

attn: AVPLAN

3619 Stonehaven Court

Orlando, Florida  32817

Please use the order form at the end of the documentation.

I will send you information on product registration and support.

AVPLAN development has been slow.  I hope that I can continue to

improve on the project.  I appreciate any contributions in the

form of database supplements and can try to provide you with

expanded database files in return.  Your comments about the

program and discovered program bugs are appreciated.

In rewriting AVPLAN version 2.2 to a structured basic for

compilation, I utilized Thomas Hanlin's ADVBAS.LIB v4.0.  Many of

the improvements made in this version rely on these routines.  I

am sincerely grateful to Mr. Hanlin for writing and circulating

these routines for general use.

I would like also, to acknowledge those people who have been

instrumental in helping to develop AVPLAN.  My wife and daughter

have provided unlimited support and patience in allowing me the

time to pursue this project.  Allen McGraw has spent many hours

helping to beta test the program and has encouraged many of the

upgrades.  Richard Wiederhold helped to develop much of my early

knowledge of Basic.  Rich has continued to the support the

project with his recommendations and editing talent.

14

DCM ENTERPRISES

attn: AVPLAN

3619 Stonehaven Court

Orlando,  Florida  32817

ORDER FORM

Name: _____________________________________________________

Address: _____________________________________________________

State: ___________________

Zip: ___________________

Home Phone: (   )     -

Bus Phone: (   )     -

Pilot:  yes    no         Grade: __________________

Comments or Suggestions: ________________________________________

_________________________________________________________________

_________________________________________________________________

There are three levels of registration listed below:

specify type of disk for level 2 and 3 -  (3.5)  or  (5.25)

number of  _____  x 10.00 (level 1)     __________

copies     _____  x 15.00 (level 2)     __________

_____  x 20.00 (level 3)     __________

Total    __________

Level 1 - Program registration only, with notification of major

updates

Level 2 - Level 1, with program disk and database

Level 3 - Level 1, 2, with a bound manual and database listing

o  Please send check or money order made out to Dale C. Metz

o  If you are interested in receiving information only, send

this form with a self addressed stamped envelope

o  If your copy of AVPLAN was purchased at a retail outlet,

please send this form with a copy of your purchase receipt so

that you can be included as a registered user. (notification

of updates)

15