FODTRACK Satellite Antenna Tracker

==================================

Version 1.2

User Manual

===========

Introduction:

=============

FodTrack is a simple, straightforward program intended to control an azimuth-

elevation rotator like the Yaesu-Kenpro 5400/5600, for any kind of satellite

orbits.

FodTrack runs in foreground, continuously displays what it is doing, uses no

special system resources, and is so simple to use that you will not have any

trouble trying to figure out what is going wrong when the silly satellites seem

to go in another direction than your antenna...

The rotor can be connected to any parallel port. LPT2 is a good candidate, as

it is often present and seldom used. Nonstandard parallel ports are welcome,

just you need to know the address. No IRQ is needed.

FodTrack is best suited for those setups where a single PC runs a multitasking

environment, like DESQview, with the satellite software in one window, a BBS in

another, and your favorite game in the third. Just open a fourth window and put

FodTrack there, it will be happy.

You can also use an independent computer for FodTrack. But you cannot run it in

background. If you want background operation, there are other programs available

for you.

FodTrack implements rotor flipping, so the satellites will not run against your

rotor's end stop. Rotor stop position can be south or north.

It can run under manual or automatic control. In automatic mode, a skeduler like

WA2N's SatSked can provide the commands.

Copyright:

==========

FodTrack is free for noncommercial use. If you want to reward me somehow, write

a piece of useful software and upload it!

Disclaimer:

===========

FodTrack is provided without any guarantee that it will really do anything of

all the nice stuff this document says. But please, if you find a bug, tell me,

so I can fix it for the next version.

Error handling is not yet very complete. Be careful.

Setting it up:

==============

This is VERY easy. Copy the FODTRACK files to a directory of your choice. You

can also maintain the kepfile there, but if you prefer you can use a kepfile at

some other place.

Edit the FODTRACK.CFG file to reflect conditions at your station. The file

explains itself.

If you will run the program from somewhere else, you need a path to that

directory.

The kepfile must be in 2-line format, with the satellite name appearing above

each 2-line block. Title lines are no problem. They are not needed, but they do

not disturb.

Running it:

===========

Execute FODTRACK.EXE. The program will come up, read its configuration file, and

then it will read the command file (FODTRACK.CMD). The default command file says

"NONE", so the program will stay idling. That's a good time to look around the

screen:

You will see a status display, which says what the program is doing. It can be

idling, waiting for a satellite, tracking it, guessing if it is convenient to

flip the antenna over, or calibrating the rotor..

There is a nice clock, ticking away your valuable seconds; also there is a line

telling you that the satellite selection is automatic, and two big windows

without any data in them. They will come to live when you select a satellite.

There's also a small reminder for the commands you have available. Please note

that these commands are not available all the time! For example, during

automatic tracking you cannot manually select a satellite, and during flipping

determination the keyboard is dead.

Now let's play a bit: Type the letter m to get the program in manual mode, then

t to start tracking a satellite. The program will ask you which satellite you

want. Enter it's name exactly as it appears in the kepfile, otherwise FodTrack

will be very unhappy with you. For example, type KO-23.

The program reads the kepfile, and displays KO-23's keps in the proper window.

If you haven't disabled flipping, it begins looking into it's crystal ball, to

see if the bird will run against your rotor stop on the next pass. If so, it

will tell you after a while that the antenna will be "flipped". If not, it will

be on the "normal" side.

After flipping determination is complete, the program will start tracking the

bird. Every second, if the computer is fast enough, the position is updated.

While the satellite is below -2 degrees elevation, no data is sent to the rotor.

When the great moment arrives, the bird is at -2 degrees, the program will

start sending target position data to your rotor, indicating so in the rotor

status display.

If the rotor is not flipped, azimuth and elevation on the rotor are the real

ones. If the program had to flip the antennas, azimuth is 180 degrees shifted

and elevation starts backwards from 180 degrees.

When the satellite goes below the horizon, the rotor output is freezed, while

the program calculates flipping for the next pass and then starts waiting for

the bird.

At any time except during flipping determination, you can stop tracking a

bird using the s command, or quit using q. If you press a, the program will go

back into automatic mode, reading the command file and doing whatever it

tells.

Rotor Interface:

================

The program generates target position data as 8 bits for each azimuth and

elevation, using one of the printer control lines (pin 14) to tell if azimuth

or elevation is meant. 0 is az, 1 is el.

The schematic diagram of the interface is provided as a printer dump file. Two

versions are provided: FODTRACK.DOT should work with most 9-pin dot matrix

printers, while FODTRACK.INK is for HP inkjet printers. Just copy the file to

your printer using COPY /B FODTRACK.DOT PRN. If you cannot print it, let me

know, and I'll try to sent you a copy in some other format. Please specify your

printer. If you can provide me some program that captures printer output and

generates a GIF or PCX image from it, it would be most welcome.

Please refer to the schematic for this explanation:

A dual DA converter (PM7226 or similiar) is directly connected to the parallel

port. The port's strobe output is wired to the DA's chip select input, pin 14

to the output select pin, and the 8 data bits to inputs 0 to 7.

The analog outputs are compared to the rotor position output using differential

amplifiers, the output of this comparison beeing fed to two window comparators

driving the motors. The value of the (presently) 1M resistors defines the

rotor's dead range, so you can adjust it by modifying these values. Lower

resistance values cause the rotor to move in bigger steps, reducing wear and

tear, but worsening pointing accuracy. If you have stability problems

(oscillations), lower these resistances.

I mounted this interface inside the G-5400B controller. I removed the 8-pin

DIN connector, used the cables to connect the interface, and also I used the

hole left by the connector to bring out the cable to the parallel port and

the enable switch.

If you don't want to modify the rotor controller, you can mount the circuit in

a separate box, and use the 8-pin connector to join the controller and the

interface. The only problem is that in this case you have no access to the

proper power supply lines. So you must use the only power provided on the

8-pin plug: unregulated 14 Volts. Use a simple zener regulator to limit the

voltage for the 12 Volt line. This line does not need to be well regulated,

but it must remain between 10 and 15 Volts, so the unregulated output from the

G-5400B should not be connected directly, or it may put the PM7226 on the

endangered species list. The 6 Volt line, on the other hand, needs good

regulation. Use a 7806 regulator here.

As you can see, the loop is closed outside the PC, in hardware. This reduces

the processing load for FodTrack. The program has no means to check if a rotor

interface is connected, so you can start it up without one.

You can copy my interface, or you can design your own one. My one works well,

but it uses the PM7226, which is not very easy to get in some countries. Any

dual 8-bit DA converter should do the job as a replacement for the PM7226, but

many of them need more power supplies, and/or have current mode output,

requiring a more complicated circuit to use them.

The program provides a calibration function which allows you to adjust your

rotor pots for correct ranging.

This sequence is valid for my interface circuit, and a Yaesu 5400 rotor:

- Assemble and install the interface, set everything up, load the program.

- Calibrate the rotor's full scale adjustments according to the rotor's manual.

- Type c to get into the calibration routine.

- Enter 255 for both azimuth and elevation.

- Calibrate the rotor's output voltage adjustments to such positions that the

rotor stays just before it's end stops (or limit switches) in both axes.

- Enter 0 for both azimuth and elevation.

- Check that the rotor turns around completely in both axes, without running

against the limits. If it does run against a limit, something is wrong...

- Play around entering values of your choice, and make sure the rotor reacts

correctly (compare the meters to what FodTrack says).

Automation:

===========

For automatic tracking of multiple satellites, configure your skeduling program

in such a way that it writes a FODTRACK.CMD file into the FodTrack directory,

containing the name of the satellite to track, exactly as it appears in the kep

file. This CMD file should be written two or three minutes before the start of

the pass, to give FodTrack enough time to do the crystal ball business and

preset the antennas.

After the pass, the skeduling system should write a CMD file saying NONE, so

FodTrack stops.

FodTrack checks the CMD file continuously, so it is mandatory to have a disk

caching program like SMARTDRIVE, or some hardware cache. Otherwise your hard

disk will complain.

In case of trouble:

===================

If you have some trouble with FodTrack, tell me, so I can fix the bugs for the

next version.

To contact me, send a message on KO-23, KO-25, or AO-16. You can also try

through the terrestrial packet net, if my program shook your satellite antennas

to pieces. The address is XQ2FOD@XQ2FOD.SER.CHL.SA.

On Internet, you can reach me at mmornhin@eso.org. Be patient, I have access to

internet only every second week, because of my working schedule.

Is there anyone around who likes to use snailmail? If so,

Manfred Mornhinweg

Radio Club Coquimbo

Casilla 381

Coquimbo

Chile

A last word:

============

I'm not a professional programmer, so you are allowed to smile about my program

and me...