������������������������������ͻ

�	PROGRAMMER'S REFERENCE    �

������������������������������ͼ

The Genius Scanner (GeniScan) enables you to use scanner unit to catch the

picture and send data through a software program.  The various features

could be made for the picture, such as cut, paste, merge, overlay and edit.

In order for you to interface scanner with your application program, the

following information on the GeniScan's interface card has been provided.

Technical data for : GS-4500

First Release      : February 25, 1989

Latest Revision    : April 24, 1989

Created by	      : Kun Ying Enterprise

1. VIDEO DATA RECEIVE METHODS: DMA1, DMA3, IRQ1, IRQ3

2. ADDRESS GROUP: 4 GROUP

GROUP 1: 272, 273, 27A, 27B

GROUP 2: 2E1, 2E2, 2E3, 2E4

GROUP 3: 371, 372, 373, 374

GROUP 4: 3E1, 3E2, 3E3, 3E4

3. REGISTERS:

�������������������������������Ŀ

� GROUP � VDP � SP  �CTRP �CLRP �

�������������������������������Ĵ

�   1	� 272 � 273 � 27A � 27B �    VDP : VIDEO DATA PORT

�������������������������������Ĵ

�   2	� 2E1 � 2E2 � 2E3 � 2E4 �    SP  : STATUS PORT

�������������������������������Ĵ

�   3	� 371 � 372 � 373 � 374 �    CTRP: CONTROL PORT

�������������������������������Ĵ

�   4	� 3E1 � 3E2 � 3E3 � 3E4 �    CLRP: COUNT CLEAR PORT

���������������������������������

4. VIDEO DATA PORT: (READ ONLY)

7	  6	 5	4      3      2      1	    0

�������������������������������������������������������Ŀ

� VD7  � VD6  � VD5  � VD3  � VD2  � VD2  � VD1  � VD0	�

���������������������������������������������������������

VDX: BIT MAP VIDEO DATA

O: BLACK

1: WHITE

5. STATUS PORT: (READ ONLY)

7	  6	 5	4      3      2      1	    0

�������������������������������������������������������Ŀ

� AR   � IRQ5 � AU   � IRQ3 � DMA3 � WG   � DMA1 � IRQ	�

���������������������������������������������������������

a. DPI DETECT:

��������������������Ŀ

� AR � AU � WG � DPI �

��������������������Ĵ

� 0  � 0  � 0  � 400 �

��������������������Ĵ

� 0  � 0  � 1  � 300 �

��������������������Ĵ

� 0  � 1  � 0  � 200 �

��������������������Ĵ

� 0  � 1  � 1  � 100 �

��������������������Ĵ

� 1  � X  � X  � XXX �

����������������������

X: Don't care

b. DMA & IRQ CONFIGURATION SETTING:

0 is ON, 1 is OFF.

������������������������������������Ŀ

� IRQ5 � IRQ3 � DMA3 � DMA1 � SELECT �

������������������������������������Ĵ

�  0   �  1   �  1	 �  1	�  IRQ5  �

������������������������������������Ĵ

�  1   �  0   �  1	 �  1	�  IRQ3  �

������������������������������������Ĵ

�  1   �  1   �  0	 �  1	�  DMA3  �

������������������������������������Ĵ

�  1   �  1   �  1	 �  0	�  DMA1  �

��������������������������������������

C. IRQ: IRQ STATUS BIT. 1 MEANS IRQ REQUEST

6. CONTROL PORT: (WRITE ONLY)

7	  6	 5	4      3      2      1	    0

�������������������������������������������������������Ŀ

� C3   � C2   � C1   � C0   �	   �	  �	 � EN	�

���������������������������������������������������������

a. EN: SCANNER POWER CONTROL BIT.

0 -- TURN OFF SCANNER POWER

1 -- TRUN ON SCANNER POWER

b. COUNT NUMBER SELECT: SCAN IN DATA (PIXELS PER LINE)

����������������������������������������Ŀ

� C3 � C2 � C1 � C0 � COUNT � REMARK	  �

����������������������������������������Ĵ

� 0  � 0  � 1	� 1  � 3648  � RESERVED   �

����������������������������������������Ĵ

� 1  � 0  � 0	� 0  � 1264  � 300 DPI	  �

����������������������������������������Ĵ

� 1  � 0  � 0	� 1  � 2544  � RESERVED   �

����������������������������������������Ĵ

� 1  � 0  � 1	� 0  �	840  � 200 DPI	  �

����������������������������������������Ĵ

� 1  � 0  � 1	� 1  � 1696  � RESERVED   �

����������������������������������������Ĵ

� 1  � 1  � 1	� 0  � 1648  � 400 DPI	  �

����������������������������������������Ĵ

� 1  � 1  � 1	� 1  �	424  � 100 DPI	  �

������������������������������������������

7. COUNT CLEAR PORT: (WRITE ONLY) WRITE ANY VALUE TO THIS PORT WILL CLEAR

BIT COUNTER.

***************************************************************************

;

;

;CONTROL SCANNER AND DMA ROUTINE

.

.

.

.

call	scan_resolution       ;Initial scanner resolution

.			      ;user define dpi

.

.

.

.

set_dma_routine:

call	check_dma_bank	      ;auto detect DMA channel

jnc	go_on

mov	scan_ok, 00	      ;return scan error code

jmp	scan_end	      ;DMA error

go_on

;

call	check_io_dpi	      ;Option,

;			      ;auto detect scanner dpi switch

jnc	go_on_1

mov	scan_ok,00	      ;return error code

jmp	scan_end	      ;scanner dpi switch check error

;

go_on_1:

mov	dx.ctr1_port_1	      ;scanner I/O address

mov	al,io_value	      ;define count number (pixels)

out	dx,al

;

call	dma_pro 	      ;set dma routine

;

move	dx,08

in	al,dx		      ;dma reached a terminal count

jmp	short d888

d888:

test	al,channel_bit

jz	temp_mouse

jmp	next_64k	      ;Have another 64K memory?

;			      ;or receive next line data

temp_mouse:

xor	bx,bx

mov	ax,03

int	33h

test	bx,03		      ;click mouse button to stop

;scan

jz	chk_kb

jmp	scan_end	      ;Stop

chk_kb:

mov	dl.0ffh

mov	ah,06

int	21h		      ;strike any key to stop scan

;

je	no_key_strok

jmp	scan_end	      ;stop scan

no_key_strok:

mov	dx,0ch

xor	ax,ax

out	dx,al		      ;clear DMA flip flop

jmp	short c000

c000:

mov	al,channel

mov	dx,ax

shl	dx,1

in	al,dx

jmp	short c111

c111:

mov	bl,al

in	al,dx

jmp	short c222

c222:

mov	bh,al		      ;read current DMA address-bx

.

.

call	display image

.

.

.

.

.

scan_end

mov	ax,dma_page

mov	dx,dma_port

out	dx,al		      ;set dma page

jmp	short dmaal

;

dmaal:

mov	al,channel

add	al,04

mov	dx,0ah

out	dx,al		      ;set channel mask bit

jmp	short dmaa2

;

dmaa2:

mov	al,channel

add	al,44h

mov	dx,0bh

out	dx,al		      ;set channel mode

jmp	short dmaa3

;

dmaa3:

mov	dx,ctrl_port

mov	al,io_value

and	al,0feh

out	dx,al		      ;turn off scanner

.

.

.

scan_resolution proc

xor    ax,ax

cmp    dpi,200h      ;200 DPI

jne    check_300dpi

mov    io_value,0a1h

mov    resolution,69h		      ;840 pixels, 69h bytes

mov    end_offset,0fff0h

jmp    end_dpi

check_300dpi:

cmp    dpi,300h      ;300 DPI

jne    check_300dpi

mov    io_value,081h

mov    resolution,09eh		      ;1264 pixels, 09eh bytes

mov    end_offset,0ff84h

jmp    end_dpi

check_400dpi:

cmp    dpi,400h      ;400 DPI

jne    check_400dpi

mov    io_value,031h

mov    resolution,0ceh		      ;1648 pixels, 0ceh bytes

mov    end_offset,0ffe4h

jmp    end_dpi

check_100dpi:

cmp    dpi,100h      ;100 DPI

je     check_100dpi_temp

stc

ret

check_100dpi_temp:

mov    al,0f1h

mov    resolution,35h		      ;424 pixels, 035h bytes

mov    end_offset,0ffe4h

end_dpi:

clc

ret

scan_resolution endp

check_dma_bank proc

clc

mov	cx,04

xor	si,si

check_bank_again:

mov	dx,work ptr status_port_1[si]

in	al,dx

cmp	al,0ffh

je	not_this_bank_group

and	al,0ah		      ;mask value 00001010b

;

cmp	al,08h		      ;bit 1 is 0 use DMA 1

je	DMA_channel_1

cmp	al,02h		      ;bit 3 is 0 use DMA 3

je	DMA_channel_3

;

not_this_bank_group:

inc	si

inc	si

dec	cx

jnz	check_bank_again

stc

ret

;

DMA_channel_1:

mov	channel,01

mov	dma_port,83h

mov	channel_bit,02	      ;set channel 1 parameter

jmp	set_bank

DMA_channel_3:

mov	channel,03

mov	dma_port,82h

mov	channel_bit,08	      ;set channel 3 parameter

set_bank:

mov	bx,word ptr ctrl_port_1[si]

mov	ctrl_port,bx

mov	bx,word ptr status_port_1[si]

mov	status_port,bx

;

mov	dx,ctrl_port

mov	al,io_value

out	dx,al		      ;turn on scanner

;

inc	dx		      ;count clear port

out	dx,al		      ;clear DMA bit count

;

clc

ret

check_dma_bank endp

check_io_dpi   proc			      ;check scanner dpi switch

mov	dx,status_port	      ;get I/O status port address

read_again:

in	al,dx		      ;wait until AR is 1

test	al,80h

jz	read_again

read_again1:

in	al,dx		      ;wait until AR is 0

test	al,80h

jnz	read_again1

;

and	al,0a4h

;

cmp	al,24h

je	100_dpi

cmp	al,20h

je	200_dpi

cmp	al,04h

je	300_dpi

cmp	al,00h

jne	next_page

mov	io_value,0e0h	      ;400 dpi 105mm 1648 pixels

mov	end_offset,0ffe4h     ;end address of 64k

jmp	set_dma

100_dpi

mov	io_value,0f0h	      ;100 dpi 105mm 424 pixels

mov	end_offset,0ffe4h     ;end address of 64k

jmp	set_dma

200_dpi:

mov	io_value,0a0h	      ;200 dpi 105mm 840 pixels

mov	end_offset,0fff0h     ;end address of 64k

jmp	set_dma

300_dpi:

mov	io_value,80h	      ;300 dpi 105mm 1264 pixels

mov	end_offset,0ff84h     ;end address of 64k

;

set_dma:

ret

check_io_dpi   endp

dma_pro        proc	NEAR

mov	ax,dam_page

mov	dx,dma_port

out	dx,al		      ;set dma page

jmp	short dma1

;

dma1:

mov	ax,channel

add	al,04

mov	dx,0ah

out	dx,al		      ;set channel mask bit

jmp	short		      dma2

;

dma2:

mov	ax,channel

add	al,44h

mov	dx,0bh

out	dx,al		      ;set channel mode

jmp	short dma3

;

dma3:

mov	dx,0ch

out	dx,al		      ;clear flip flop

jmp	short dma4

dma4:

mov	ax,channel

mov	dx,ax

xor	al,al

sh1	dx,1

out	dx,al

jmp	short dma5

dma5:

out	dx,al

jmp	short dma6	      ;set dma start address from 0000

dma6:

mov	dx,0ch

out	dx,al		      ;clear flip flop

jmp	short dma7

dma7:

mov	ax,channel

mov	dx,ax

sh1	dx,1

inc	dx

mov	ax,end_offset

dec	ax

out	dx,al		      ;high byte of end dma offset

jmp	short dma8

dma8:

mov	al,ah

out	dx,al		      ;low byte of end dma offset

jmp	short dma9	      ;set dma count number

dma9:

mov	dx,ctrl_port

inc	dx

out	dx,al		      ;clear interface card receive counter

jmp	short dma9_1	      ;write any value

;

dma9_1:

mov	dx,status_port	      ;Get video buffer port

dec	dx

in	al,dx		      ;reset dma request

jmp	short dma9_2

;

dma9_2:

mov	ax,channel

mov	dx,09

out	dx,al		      ;reset dma channel

jmp	short dma10

;

dma10:

inc	dx

out	dx,al		      ;clear masked channel

dma_delay:

jmp	short dma11

dma11:

ret

dma_pro        endp

;SCANNER I/O ADDRESS

ctrl_port      dw	027ah

status_port    dw	0273h

ctrl_port_1    dw	027ah

dw	02e3h

dw	0373h

dw	03e3h

status_port_1  dw	0273h

dw	02e2h

dw	0372h

dw	03e2h