Subject: PWRSAVE.COM for the T1200

It is our pleasure to introduce the new utility PWRSAVE.COM for the T1200.  It

increases battery life. Battery life is a function of hard disk use.  Since

many applications, such as word processing, load data into RAM and carry out all

editing there without reaccessing the hard disk, it makes sense to find a way to

turn off the hard disk while not in use and thereby save power.  The intelligent

power supply of the T1200 allows for programming of operating voltage.  The new

utility takes advantages of this feature to turn off the hard disk when not in

use.

PWRSAVE.COM is described in more detail on the next page.  You can donwload this

utility from file area 1 for white machines on our BBS(+49-2101-120652 1200 8N1).

PWRSAVE.COM has the following operations:

t_	Sets the time period

-	Deactivates the power saver

+	Activates the power saver

?	Displays status

PWRSAVE.COM observes whether the hard disk has been accessed in a period defined

by the user.  If no access occurs in the defined period, say 3 minutes, then the

power to the hard disk is automatically turned off, thus reducing battery drain.

The hard disk is turned on again automatically when the hard disk is reaccessed.

When setting the time period, consideration should be given to the fact that

turning the hard disk on and off frequently is also energy intensive. Therefore,

care should be taken not to use the utility with applications where disk access

is frequent.

27.05.88                                        Helmut Kopel