;		  Modification of the Z-100 MS-DOS BIOS

;		   to support four 5.25" floppy drives

;

;			By John Stetson 21-Apr-85

;

;	This file contains the modifications to the Z-100 MS-DOS BIOS

;	(for both MS-DOS V2.13 and MS-DOS V3.10) which support the

;	use of either three or four 5" inch floppy disk drives.

;

;	As modified, drives A and B are 96 tpi and drives C and D

;	are 48 tpi.  The 8" drives at C and D have been eliminated

;	in order to retain the standard Winchester partitions at E-G.

;

;	The old and new disk configurations are given below:

;

;	Drive	Old		New

;

;	A:	5" 48 tpi	5" 96 tpi

;	B:	5" 48 tpi	5" 96 tpi

;	C:	8"		5" 48 tpi

;	D:	8"		5" 48 tpi

;	E:	Winchester	Winchester

;	F:	Winchester	Winchester

;	G:	Winchester	Winchester

;	H:	Winchester	Winchester

;

;	The following BIOS files must be edited and reassembled:

;

;	DEFDSK.ASM, BINIT.ASM, BBLKDEV.ASM, BDSKTB.ASM

;

;	This file contains those portions of the above files

;	which have been modified.  To implement the modifications,

;	you should obtain a hard-copy listing of this file on

;	a printer, make the indicated changes (most modified lines

;	are indicated by the identifier 'JFS') to the BIOS source

;	code files provided as part of the MS-DOS Programmer's

;	Utility Pack, and reassemble and relink the BIOS to make

;	a new IO.SYS file with which you will replace the original

;	MS-DOS IO.SYS file.  This process can take a considerable

;	amount of time and effort due to a number of mistakes in

;	the MS-DOS V2 Programmer's Utility Pack in the directions

;	for reassembling the BIOS!  Make sure you can correctly

;	assemble the BIOS without errors and that the IO.SYS file

;	you produce works correctly as-is before attempting these

;	modifications.

;

;	If you are using a version of MS-DOS V2.X later than 2.13

;	for the Z-100 (i.e. Version 2.21), you will either have to

;	return to using 2.13 (with the appropriate patches to

;	correct the serial I/O bug, etc. or obtain the source code

;	for your version of the BIOS (it can be done by comparing

;	and disassembling the two IO.SYS files and then editing the

;	2.13 source to update it to 2.21).

;

;	After making the new IO.SYS file, format a new bootable

;	5" floppy disk and replace the existing IO.SYS file

;	with the new one (use the FLAGS.COM program to remove

;	the flags from the old IO.SYS file, delete it and copy

;	the new one to the disk - note - IO.SYS MUST be the

;	first file on the disk).

;

;	Replace the existing 2 drive cable with either a three

;	or four drive cable, set the drive programming plugs,

;	and you should be able to access either 3 or 4 5" drives.

;

;	All is not sweetness and light, however!  The Z-100

;	FORMAT program is hard-coded for drives A and B as 5"

;	drives and C and D as 8" and E-G as Winchester partitions.

;	You can not format any type of 5" drive which is physically

;	installed (via the drive programming plug) as drive C or D.

;

;	To rub salt into the wound, the FORMAT program also reads

;	the 8 position DIP switch on the Z-207 floppy disk controller

;	card to determine whether the 5" drive being formatted is

;	48 tpi or 96 tpi.  The BIOS also looks at this switch under

;	certain circumstances (so it knows when to double step when

;	reading a 48 tpi disk in a 96 tpi drive).  You must place

;	this switch in the proper position when formatting disks

;	in drives A and B and when reading 48 tpi disks in 96 tpi

;	drives.  You will be able to read, write and format 96 tpi

;	disks in drives A and B, read 48 tpi disks in drives A and

;	B, and read and write 48 tpi disks in drives C and D - you

;	will not be able to format 48 tpi disks.  Note - this may

;	be possible by changing the software and hardware so that

;	A=96 tpi, B=48 tpi, C=96 tpi, D=48 tpi, but I haven't tried

;	this.  I have soldered a twisted pair of wires to the switch

;	on the Z-207 card and connected them to a toggle switch

;	mounted on the rear of the computer to make it easier to

;	flip the switch.

;

;	BIOS Source Code Modifications

;

;	DEFDSK.ASM

;

;	(lines omitted)

;

;  Define the disk info block (one is needed for each drive)

;

;

MAXDSK		EQU 15		; Maximum number of disks

;

;MAXDSK5	EQU 2		; Maximum 5 inch drives (0-1)           JFS

MAXDSK5	        EQU 4		; Maximum 5 inch drives (0-3)           JFS

;

;MAXDSK8	EQU 4		; Maximum 8 inch drives (2-3)           JFS

MAXDSK8	        EQU 4		; Maximum 8 inch drives (none)          JFS

;

MAXDSKW		EQU 8		; Maximum winchester drive (4-7)

;

;	(rest of file omitted)

;

;	BINIT.ASM

;

;	(lines omitted)

;

; Now map non-existant drives into imaginary ones

;

MOV	AX,WORD PTR EXSTDRV ; Get bit vector of existance

AND	AX,1111B	; 0FFFFH SHR (16-MAXDSK5) ; Only look at 5"

XOR	BX,BX		; Get index zero in DSK vector

MOV	CX,MAXDSK5	; Get count

CALL	MAPDRV		; Map those drives

;

;       MOV	AX,WORD PTR EXSTDRV ; Get bit vector of existance       JFS

;	MOV	CX,MAXDSK5	; Skip over 5 inchers

;	SHR	AX,CL

;       AND	AX,1111B	; look at 8 inchers

;	MOV	BX,MAXDSK5*2	; Get word index into DSK vector

;	MOV	CX,MAXDSK8-MAXDSK5 ; Get count

;	CALL	MAPDRV		; Map those drives                      JFS

;

;	(rest of file omitted)

;

;	BBLKDEV.ASM

;

;	(lines omitted)

;

DSK0_MCHK1B:

POPF

;

;	Check status of drive

;

MOV	AL,BYTE PTR SRH_UNIT[BX]	; Get unit number

MOV	CS:DSKPR+DSKPR_DRIVE,AL		; Set it in

PUSH	DS

PUSH	BX

PUSH	DI			; Save current pointers

MOV	BX,CS

MOV	ES,BX

MOV	DS,BX

MOV	BX,OFFSET DSKPR		; ES:BX pointer to dskpr

MOV	AL,DSK_STATUS		; Get status

CALL	DSK			; Call it

POP	DI

POP	BX

POP	DS			; AH = status, 'CY' if not ready

JC	DSK0_MCKNR		; Drive is not ready

;

CMP	BYTE PTR SRH_UNIT[BX],MAXDSK5	; < 4 if 5" drive       JFS

JNC	DSK0_MCK0		; If not, 8" drive              JFS

JMP	NEAR PTR DSK0_MCK1	; Was < 4, 5"                   JFS

;

;       (lines omitted)

;

DSK0_BPB:

;

;	compute SI as pointer to proper BPB

;

MOV	AL,SRH_UNIT[BX]		; AL = unit number

CBW

SHL	AX,1			; Make into word offset

MOV	DI,AX

MOV	DI,CS:WORD PTR DSK_TPTR[DI]	; Get disk table pointer

;

CMP	BYTE PTR SRH_UNIT[BX],MAXDSK5	; < 4 if 5" drive       JFS

JNC	DSK0_BPB0		; If not, 8" or winchester      JFS

JMP	NEAR PTR DSK0_BPB1	; Was < 4, 5"                   JFS

;

;       (lines omitted)

;

;*	BPB Vector table

;

;	1 entry per unit, the largest entry for each unit

;

;

BPBVEC:

;   REPT MAXDSK5

DW	BPB3E			; Drive A: 5" 96 tpi/ds ext.    JFS

DW	BPB3E			; Drive B: 5" 96 tpi/ds ext.    JFS

DW	BPB1E			; Drive C: 5" 48 tpi/ds ext.    JFS

DW	BPB1E			; Drive D: 5" 48 tpi/ds ext.    JFS

;   ENDM

;

;   REPT MAXDSK8-MAXDSK5                                                JFS

;       DW	BPB2			; Drive C:-D: 8" ds/dd          JFS

;   ENDM                                                                JFS

;

;	(rest of file omitted)

;

;	BDSKTB.ASM

;

;	(lines omitted)

;

DSK_TPTR	LABEL	WORD

;

DW	OFFSET DISK0

DW	OFFSET DISK1

;

DW	OFFSET DISK2            ; Changed from 8" to 5"         JFS

DW	OFFSET DISK3            ; Changed from 8" to 5"         JFS

;

DW	OFFSET DISK4

DW	OFFSET DISK5

DW	OFFSET DISK6

DW	OFFSET DISK7

DW	-1			; Mark end of table

;

SPDISK	DB	DSK_SIZE DUP(0FFH)	; Initialized to an invalid value

;

;       The default drive step times were changed from                  JFS

;       30ms to 6ms for all 4 of the 5" floppy drives                   JFS

;

;   5.25 inch Unit 0

;

;

DISK0	LABEL	BYTE

ERRNZ	DISK0,DSK_STA

DW	0			; Status

ERRNZ	DISK0,DSK_TYPE

DB	DSK_TZ2075		; Z-207 type disk

ERRNZ	DISK0,DSK_LTRK

DB	0			; Last track

ERRNZ	DISK0,DSK_LOPT

DB	DSK_OUK			; Last operation is unknown

ERRNZ	DISK0,DSK_FLAG

DB	DSK_FDS+DSK_FDC+DSK_FSL	; Double sided disk

ERRNZ	DISK0,DSK_SEL

DB	CONDSEN+0+CONPC		; Select disk 0, no precomp

ERRNZ	DISK0,DSK_RS

DB	FDCRST+FDFHLB+FDFS6	; Restore(home) command 	JFS

ERRNZ	DISK0,DSK_SPHI

DB	FDCSTI+FDFHLB+FDFS6+FDFUTR ; Step in command		JFS

ERRNZ	DISK0,DSK_FMT

DB	FDCWRT+FDFDLF		; Format(write track) command

ERRNZ	DISK0,DSK_RD

DB	FDCRDS+FDFSLF		; Read command

ERRNZ	DISK0,DSK_WR

DB	FDCWRS+FDFSLF		; Write command

ERRNZ	DISK0,DSK_SK

DB	FDCSEK+FDFHLB+FDFS6	; Seek command			JFS

ERRNZ	DISK0,DSK_SERR

DW	0			; # of soft errors

ERRNZ	DISK0,DSK_MAXT

DB	80			; Number of tracks on disk

ERRNZ	DISK0,DSK_NRETRY

DB	1			; Number of retrys

ERRNZ	DISK0,DSK_SPT

DB	8			; Sectors per track

ERRNZ	DISK0,DSK_BPS

DW	512			; Bytes per sector

ERRNZ	DISK0,DSK_BPWT

DW	6437			; Bytes per write track

ERRNZ	DISK0,DSK_BPRT

DW	6437			; Bytes per read track

ERRNZ	DISK0,DSK_DELAY

DW	400			; 400 instr = 1.5mls

ERRNZ	DISK0,DSK_LDELAY

DW	8000			; 8000 instr = 30mls

ERRNZ	DISK0,DSK_PORT

DW	Z207A			; Base Port number

ERRNZ	DISK0,DSK_RDT

DB	FDCRDT+FDFDLF		; Read track command

ERRNZ	DISK0,DSK_IMGFLG

DB	DSKIF_DV		; Drive exits

ERRNZ	DISK0,DSK_TDSEL

DW	100			; Time to wait before deselect

ERRNZ	DISK0,DSK_NAME

DB	'A'			; Drive A:

ERRNZ	DISK0,DSK_TIMOUT

DW	0			; Current timeout

ERRNZ	DISK0,DSK_SIZE

;

;

;  5.25 inch, Unit 1

;

;

DISK1	LABEL	BYTE

ERRNZ	DISK1,DSK_STA

DW	0			; Status

ERRNZ	DISK1,DSK_TYPE

DB	DSK_TZ2075		; Z-207 type disk

ERRNZ	DISK1,DSK_LTRK

DB	0			; Last track

ERRNZ	DISK1,DSK_LOPT

DB	DSK_OUK			; Last operation is unknown

ERRNZ	DISK1,DSK_FLAG

DB	DSK_FDS+DSK_FDC+DSK_FSL	; Double sided disk

ERRNZ	DISK1,DSK_SEL

DB	CONDSEN+1+CONPC		; Select disk 1, no precomp

ERRNZ	DISK1,DSK_RS

DB	FDCRST+FDFHLB+FDFS6	; Restore(home) command 	JFS

ERRNZ	DISK1,DSK_SPHI

DB	FDCSTI+FDFHLB+FDFS6+FDFUTR ; Step in command		JFS

ERRNZ	DISK1,DSK_FMT

DB	FDCWRT+FDFDLF		; Format(write track) command

ERRNZ	DISK1,DSK_RD

DB	FDCRDS+FDFSLF		; Read command

ERRNZ	DISK1,DSK_WR

DB	FDCWRS+FDFSLF		; Write command

ERRNZ	DISK1,DSK_SK

DB	FDCSEK+FDFHLB+FDFS6	; Seek command			JFS

ERRNZ	DISK1,DSK_SERR

DW	0			; # of soft errors

ERRNZ	DISK1,DSK_MAXT

DB	80			; Number of tracks on disk

ERRNZ	DISK1,DSK_NRETRY

DB	1			; Number of retrys

ERRNZ	DISK1,DSK_SPT

DB	8			; Sectors per track

ERRNZ	DISK1,DSK_BPS

DW	512			; Bytes per sector

ERRNZ	DISK1,DSK_BPWT

DW	6437			; Bytes per write track

ERRNZ	DISK1,DSK_BPRT

DW	6437			; Bytes per read track

ERRNZ	DISK1,DSK_DELAY

DW	400			; 400 instr = 1.5mls

ERRNZ	DISK1,DSK_LDELAY

DW	8000			; 8000 instr = 30mls

ERRNZ	DISK1,DSK_PORT

DW	Z207A			; Base Port number

ERRNZ	DISK1,DSK_RDT

DB	FDCRDT+FDFDLF		; Read track command

ERRNZ	DISK1,DSK_IMGFLG

DB	DSKIF_DV		; Drive exits

ERRNZ	DISK1,DSK_TDSEL

DW	100			; Time to wait before deselect

ERRNZ	DISK1,DSK_NAME

DB	'B'			; Drive B:

ERRNZ	DISK1,DSK_TIMOUT

DW	0			; Current timeout

ERRNZ	DISK1,DSK_SIZE

;

;

;   5.25 inch Unit 2

;

;

DISK2	LABEL	BYTE

ERRNZ	DISK2,DSK_STA

DW	0			; Status

ERRNZ	DISK2,DSK_TYPE

DB	DSK_TZ2075		; Z-207 type disk

ERRNZ	DISK2,DSK_LTRK

DB	0			; Last track

ERRNZ	DISK2,DSK_LOPT

DB	DSK_OUK			; Last operation is unknown

ERRNZ	DISK2,DSK_FLAG

DB	DSK_FDS+DSK_FDC+DSK_FSL	; Double sided disk

ERRNZ	DISK2,DSK_SEL

DB	CONDSEN+2+CONPC		; Select disk 2, no precomp

ERRNZ	DISK2,DSK_RS

DB	FDCRST+FDFHLB+FDFS6	; Restore(home) command 	JFS

ERRNZ	DISK2,DSK_SPHI

DB	FDCSTI+FDFHLB+FDFS6+FDFUTR ; Step in command		JFS

ERRNZ	DISK2,DSK_FMT

DB	FDCWRT+FDFDLF		; Format(write track) command

ERRNZ	DISK2,DSK_RD

DB	FDCRDS+FDFSLF		; Read command

ERRNZ	DISK2,DSK_WR

DB	FDCWRS+FDFSLF		; Write command

ERRNZ	DISK2,DSK_SK

DB	FDCSEK+FDFHLB+FDFS6	; Seek command			JFS

ERRNZ	DISK2,DSK_SERR

DW	0			; # of soft errors

ERRNZ	DISK2,DSK_MAXT

DB	80			; Number of tracks on disk

ERRNZ	DISK2,DSK_NRETRY

DB	1			; Number of retrys

ERRNZ	DISK2,DSK_SPT

DB	8			; Sectors per track

ERRNZ	DISK2,DSK_BPS

DW	512			; Bytes per sector

ERRNZ	DISK2,DSK_BPWT

DW	6437			; Bytes per write track

ERRNZ	DISK2,DSK_BPRT

DW	6437			; Bytes per read track

ERRNZ	DISK2,DSK_DELAY

DW	400			; 400 instr = 1.5mls

ERRNZ	DISK2,DSK_LDELAY

DW	8000			; 8000 instr = 30mls

ERRNZ	DISK2,DSK_PORT

DW	Z207A			; Base Port number

ERRNZ	DISK2,DSK_RDT

DB	FDCRDT+FDFDLF		; Read track command

ERRNZ	DISK2,DSK_IMGFLG

DB	DSKIF_DV		; Drive exits

ERRNZ	DISK2,DSK_TDSEL

DW	100			; Time to wait before deselect

ERRNZ	DISK2,DSK_NAME

DB	'C'			; Drive C:

ERRNZ	DISK2,DSK_TIMOUT

DW	0			; Current timeout

ERRNZ	DISK2,DSK_SIZE

;

;

;  5.25 inch, Unit 3

;

;

DISK3	LABEL	BYTE

ERRNZ	DISK3,DSK_STA

DW	0			; Status

ERRNZ	DISK3,DSK_TYPE

DB	DSK_TZ2075		; Z-207 type disk

ERRNZ	DISK3,DSK_LTRK

DB	0			; Last track

ERRNZ	DISK3,DSK_LOPT

DB	DSK_OUK			; Last operation is unknown

ERRNZ	DISK3,DSK_FLAG

DB	DSK_FDS+DSK_FDC+DSK_FSL	; Double sided disk

ERRNZ	DISK3,DSK_SEL

DB	CONDSEN+3+CONPC		; Select disk 3, no precomp

ERRNZ	DISK3,DSK_RS

DB	FDCRST+FDFHLB+FDFS6	; Restore(home) command 	JFS

ERRNZ	DISK3,DSK_SPHI

DB	FDCSTI+FDFHLB+FDFS6+FDFUTR ; Step in command		JFS

ERRNZ	DISK3,DSK_FMT

DB	FDCWRT+FDFDLF		; Format(write track) command

ERRNZ	DISK3,DSK_RD

DB	FDCRDS+FDFSLF		; Read command

ERRNZ	DISK3,DSK_WR

DB	FDCWRS+FDFSLF		; Write command

ERRNZ	DISK3,DSK_SK

DB	FDCSEK+FDFHLB+FDFS6	; Seek command			JFS

ERRNZ	DISK3,DSK_SERR

DW	0			; # of soft errors

ERRNZ	DISK3,DSK_MAXT

DB	80			; Number of tracks on disk

ERRNZ	DISK3,DSK_NRETRY

DB	1			; Number of retrys

ERRNZ	DISK3,DSK_SPT

DB	8			; Sectors per track

ERRNZ	DISK3,DSK_BPS

DW	512			; Bytes per sector

ERRNZ	DISK3,DSK_BPWT

DW	6437			; Bytes per write track

ERRNZ	DISK3,DSK_BPRT

DW	6437			; Bytes per read track

ERRNZ	DISK3,DSK_DELAY

DW	400			; 400 instr = 1.5mls

ERRNZ	DISK3,DSK_LDELAY

DW	8000			; 8000 instr = 30mls

ERRNZ	DISK3,DSK_PORT

DW	Z207A			; Base Port number

ERRNZ	DISK3,DSK_RDT

DB	FDCRDT+FDFDLF		; Read track command

ERRNZ	DISK3,DSK_IMGFLG

DB	DSKIF_DV		; Drive exits

ERRNZ	DISK3,DSK_TDSEL

DW	100			; Time to wait before deselect

ERRNZ	DISK3,DSK_NAME

DB	'D'			; Drive D:

ERRNZ	DISK3,DSK_TIMOUT

DW	0			; Current timeout

ERRNZ	DISK3,DSK_SIZE

;

;

;	DISK2 - 8" unit 0

;

;DISK2	LABEL	BYTE

;	ERRNZ	DISK2,DSK_STA

;	DW	0			; Status

;	ERRNZ	DISK2,DSK_TYPE

;	DB	DSK_TZ2078		; Z-207 type disk

;	ERRNZ	DISK2,DSK_LTRK

;	DB	0			; Last track

;	ERRNZ	DISK2,DSK_LOPT

;	DB	DSK_OUK			; Last operation is unknown

;	ERRNZ	DISK2,DSK_FLAG

;	DB	DSK_FDS+DSK_FDC+DSK_FRS	; Double sided disk

;	ERRNZ	DISK2,DSK_SEL

;	DB	CONDSEN+0+CONDS8+CONPC	; Select disk 0, no precomp

;	ERRNZ	DISK2,DSK_RS

;	DB	FDCRST+FDFHLB+FDFS30	; Restore(home) command

;	ERRNZ	DISK2,DSK_SPHI

;	DB	FDCSTI+FDFHLB+FDFS30+FDFUTR ; Step in command

;	ERRNZ	DISK2,DSK_FMT

;	DB	FDCWRT+FDFDLF		; Format(write track) command

;	ERRNZ	DISK2,DSK_RD

;	DB	FDCRDS+FDFSLF		; Read command

;	ERRNZ	DISK2,DSK_WR

;	DB	FDCWRS+FDFSLF		; Write command

;	ERRNZ	DISK2,DSK_SK

;	DB	FDCSEK+FDFHLB+FDFVRF+FDFS30	; Seek command

;	ERRNZ	DISK2,DSK_SERR

;	DW	0			; # of soft errors

;	ERRNZ	DISK2,DSK_MAXT

;	DB	77			; Number of tracks on disk

;	ERRNZ	DISK2,DSK_NRETRY

;	DB	1			; Number of retrys

;	ERRNZ	DISK2,DSK_SPT

;	DB	8			; Sectors per track

;	ERRNZ	DISK2,DSK_BPS

;	DW	1024			; Bytes per sector

;	ERRNZ	DISK2,DSK_BPWT

;	DW	1			; Bytes per write track

;	ERRNZ	DISK2,DSK_BPRT

;	DW	1			; Bytes per read track

;	ERRNZ	DISK2,DSK_DELAY

;	DW	400			; 400 instr = 1.5mls

;	ERRNZ	DISK2,DSK_LDELAY

;	DW	8000			; 8000 instr = 30mls

;	ERRNZ	DISK2,DSK_PORT

;	DW	Z207A			; Base Port number

;	ERRNZ	DISK2,DSK_RDT

;	DB	FDCRDT+FDFDLF		; Read track command

;	ERRNZ	DISK2,DSK_IMGFLG

;	DB	DSKIF_DV		; Drive exits

;	ERRNZ	DISK2,DSK_TDSEL

;	DW	100			; Time to wait before deselect

;	ERRNZ	DISK2,DSK_NAME

;	DB	'C'			; Drive C:

;	ERRNZ	DISK2,DSK_TIMOUT

;	DW	0			; Current timeout

;	ERRNZ	DISK2,DSK_SIZE

;

;

;	DISK3 - 8" unit 1

;

;DISK3	LABEL	BYTE

;	ERRNZ	DISK3,DSK_STA

;	DW	0			; Status

;	ERRNZ	DISK3,DSK_TYPE

;	DB	DSK_TZ2078		; Z-207 type disk

;	ERRNZ	DISK3,DSK_LTRK

;	DB	0			; Last track

;	ERRNZ	DISK3,DSK_LOPT

;	DB	DSK_OUK			; Last operation is unknown

;	ERRNZ	DISK3,DSK_FLAG

;	DB	DSK_FDS+DSK_FDC+DSK_FRS	; Double sided disk

;	ERRNZ	DISK3,DSK_SEL

;	DB	CONDSEN+1+CONDS8+CONPC	; Select disk 1, no precomp

;	ERRNZ	DISK3,DSK_RS

;	DB	FDCRST+FDFHLB+FDFS30	; Restore(home) command

;	ERRNZ	DISK3,DSK_SPHI

;	DB	FDCSTI+FDFHLB+FDFS30+FDFUTR ; Step in command

;	ERRNZ	DISK3,DSK_FMT

;	DB	FDCWRT+FDFDLF		; Format(write track) command

;	ERRNZ	DISK3,DSK_RD

;	DB	FDCRDS+FDFSLF		; Read command

;	ERRNZ	DISK3,DSK_WR

;	DB	FDCWRS+FDFSLF		; Write command

;	ERRNZ	DISK3,DSK_SK

;	DB	FDCSEK+FDFHLB+FDFVRF+FDFS30	; Seek command

;	ERRNZ	DISK3,DSK_SERR

;	DW	0			; # of soft errors

;	ERRNZ	DISK3,DSK_MAXT

;	DB	77			; Number of tracks on disk

;	ERRNZ	DISK3,DSK_NRETRY

;	DB	1			; Number of retrys

;	ERRNZ	DISK3,DSK_SPT

;	DB	8			; Sectors per track

;	ERRNZ	DISK3,DSK_BPS

;	DW	1024			; Bytes per sector

;	ERRNZ	DISK3,DSK_BPWT

;	DW	1			; Bytes per write track

;	ERRNZ	DISK3,DSK_BPRT

;	DW	1			; Bytes per read track

;	ERRNZ	DISK3,DSK_DELAY

;	DW	400			; 400 instr = 1.5mls

;	ERRNZ	DISK3,DSK_LDELAY

;	DW	8000			; 8000 instr = 30mls

;	ERRNZ	DISK3,DSK_PORT

;	DW	Z207A			; Base Port number

;	ERRNZ	DISK3,DSK_RDT

;	DB	FDCRDT+FDFDLF		; Read track command

;	ERRNZ	DISK3,DSK_IMGFLG

;	DB	DSKIF_DV		; Drive exits

;	ERRNZ	DISK3,DSK_TDSEL

;	DW	100			; Time to wait before deselect

;	ERRNZ	DISK3,DSK_NAME

;	DB	'D'			; Drive D:

;	ERRNZ	DISK3,DSK_TIMOUT

;	DW	0			; Current timeout

;	ERRNZ	DISK3,DSK_SIZE

;

;	(rest of file omitted)

;

;	(End of BIOS Modifications)