Z-DOS UTILITIES

HUG P/N 885-3008-37

Introduction:   This  disk contains several utilities to aid  the

Z-DOS  user,  including two Modem communication programs,  a Menu

executive,  printer control programs,  a directory program, and a

screen dump program for IDS Prism color printers.

Requirements:   These  programs will run on any H/Z-100  computer

with Z-DOS and standard equipment.   The printer programs require

the specific printer they were intended for use with.

The following files are included on the Z-DOS Utilities Disk:

TERMZ100.COM  -- This is a modem control program patterned  after

the  Heath/Zenith  CPS program.   Features include  function  key

control  of operating commands,  emulation of different terminals

via a user alterable table, user selectable baud rate, ASCII file

upload  and down load,  and binary file upload and download  with

other computers using TERMZ100.

TERMZ100.DOC -- Instructions for TERMZ100.

TERMZ100.ASM -- The assembly source code for TERMZ100.

KTBL.ASM -- An INCLUDE file required for TERMZ100.

DTERM.COM  -- This is a "dumb terminal" modem  program,  provoded

for  those who want a modem program that does not intercept  con-

trol characters,  escape sequences, etc.  This program intercepts

only Control-E, which is used to return control to Z-DOS.

DTERM.ASM -- The assembly source code for DTERM.

MENU.COM  -- This program provides the ability to run other  pro-

grams from a screen menu.   Once installed, you only need to move

an indicator to the name of the function you want to perform with

the arrow keys,  and press the RETURN or ENTER key.  This program

could  be  of  help to your friends who are  new  users  and  are

unfamiliar with computers.   However,  a knowledgable person will

be  required to set the program up for the user.

MENU.DOC -- Instructions for setting up and using MENU.

MENU.ASM -- The assembly source code for MENU.

MENU.TBL -- This is a sample menu that can be used to demonstrate

the MENU program.  It provides menu selection of file directories

on drive A:  and B:  using DIR100 (see below),  and manual entry.

DIR100.COM  must reside on your system disk for this sample  menu

to run properly.

MENU.DAT -- This is a sample information file which is  displayed

when  you press the HELP key while using the MENU  program.   You

can  use  it as is,  or use it as a model for creating  your  own

information files.

BUILD.BAS  -- This  is a Z-BASIC program that is used  to  create

MENU.TBL  files  containing  programs that you want to  run  from

MENU.

AUTOEXEC.BAT,  XA.BAT,  XM.BAT -- These are batch files that must

reside on your system disk in order for MENU to function  proper-

ly.

DIR100.COM  -- This program prints an alphabetized list of  files

on a disk,  using the special features of the H/Z100 computer  to

produce  an easy-to-read display.   It displays up to 80 files on

the screen at a time,  showing the file name,  size in bytes, and

optionally the attributes of each file.

If  a disk has more than 80 files,  DIR100 will display the first

80 and prompt you to hit RETURN.   It will then display the  next

"page" of files,  and continue this process for a maximum of 1024

files.  You can type Control-D instead of RETURN to exit to Z-DOS

before the last page is displayed.   At the bottom of the screen,

DIR100 prints the number of files found, the total amount of disk

space  used by the files shown,  the total space actually used up

on  the  disk (in parentheses),  and the amount  of  space  free.

DIR100 takes arguments in the form

A>DIR100 [drive][filename.typ] [N][S]

where [drive] is a drive disignation,  such as A:,  B:, etc., and

can  be  left off if you want to see the current  drive's  files.

The  file name [filename.typ] can be left off to show all  files,

or  can be ambiguous (wild cards) to show groups of  files.   The

letter N causes DIR100 to suppress alphabetization and show files

in their "natural" order.   The letter S causes DIR100 to include

all  hidden and/or system files in the directory and to show  the

attribute  bits in the file name as described  below.   Here  are

some examples of the use of DIR100.

A:DIR100                      Show   all  files  on  the  current

drive (in the current user no.).

A:DIR100 A: N                 Show all files on A:,  with  alpha-

betizing suppressed.

A:DIR100 B:D*.*               Show  all  files on B:  that  start

with the letter D.

A:DIR100 A:*.SYS NS           Show  all .SYS files on  drive  A:,

not alphabetized,  including system

and/or hidden files.

A:DIR100 B:*.SYS SN           Same for drive B:.  Note that the S

can be placed before the N.

When  you use the S switch,  attributes that are set on any  file

are  shown by displaying a character of the file name that corre-

sponds to the attribute set in reverse video.   For  example,  if

the  Hidden file attribute is set,  which is the second attribute

bit,  the  second character of the file name will be  in  reverse

video.   (Note  that  while  the bits within a byte  are  usually

numbered from right to left,  the corresponding characters in the

file name are numbered from left to right.)

The  total disk space used printed at the bottom of the screen in

parentheses  may not agree with the total space used by the files

shown  even if all files shown because files are  allocated  disk

space  in blocks,  and partially filled blocks are considered  as

full blocks.

For convenience,  you can rename DIR100.COM to D.COM, which allows

you  to  display a directory simply by typing D followed  by  any

arguments required.

DIR100.ASM -- The source for DIR100.

DISKID.COM  -- This program allows you to enter a  volume  number

and  label  that  are  written  to your disk  in  a  file  called

IDENT.SYS.   The DIR100 program will print the volume number  and

label  of the disk at the top of the screen or page when you  use

them  on a disk that has an IDENT.SYS file.   Use DIR100 to  show

the  directory on this disk to see what it looks like.   The file

IDENT.SYS  created by this program has the hidden attribute  set,

and  will  not show up in the regular directory  or  with  DIR100

unless  you use the S switch.   It cannot be deleted by the ERASE

command, but you can delete it by entering

A:DISKID D

or  if  you want to remove the IDENT.SYS file on  another  drive,

enter

A:DISKID d:D

where  d:  is the designation of the other drive.   When you  run

DISKID without an argument,  it will create a new IDENT.SYS,  and

will  ask you to enter a volume number and a label.   If you want

to put escape codes in the label,  type a @ wherever you want  an

escape to be placed.  For example, to get reverse video, type @p.

DISKID.ASM -- The assembly source code for DISKID.

COLOR.COM  -- This program allows for changing the foreground and

background  colors  being displayed on the Z-100  CRT.   It  acts

exactly like the ZBASIC COLOR statement but is accessible from Z-

DOS directly.  Calling sequence is as follows:

COLOR fgr,bgr

If the parameters are ommitted,  an explaination as to its use is

displayed  on the CRT  This assembler program occupies less  than

1K and is structured as a Z-DOS "COM" file.

COLOR.ASM -- The assembly source code for COLOR.

PRISM.COM -- This utility allows for quick and easy change of IDS

Prism printer mode setup.  The program is invoked by:

PRISM <string>

Where <string> is a list of one-character commands.   If <string>

is   ommitted,   information  detailing  available  commands   is

displayed  on the console.   Commands change the various  printer

options such as ribbon color, character spacing, print modes, and

character  and  line pitch.   It is written in assembler  and  is

structured  as a 'COM' file for fast execution.   It is just over

1K in length and requires an IDS Prism printer to be of use.

PRISM.ASM -- The assembly source code for PRISM.

PRINTEK.COM  -- This  program is exactly like the  PRISM  program

above  except  that its usage is for a  PRINTEK  910/920  printer

instead of the IDS Prism printer.

PRINTEK.ASM -- The assembly source code for PRINTEK.

PSET25.COM  -- This  program  allows you to set options  on  your

H/Z25 printer before use.  To use this program, enter

A>PSET25 [argument]

with the argument being one of the following:

10 -- set 10 characters per inch.

12 -- set 12 CPI.

13 -- set 13.2 CPI.

16 -- set 16.5 CPI.

6  -- set 6 lines/inch.

8  -- set 8 lines/inch.

Run PSET25 without an argument to see a list of them.

PSET25.ASM is the source for PSET25.

PSETMX.COM  -- This  program is like PSET25,  but  for  the  MX80

printer.  The arguments it takes are

C -- set compressed printing.

D -- set double strike.

E -- set emphasized printing.

N -- set normal printing (cancel all of the above).

6 -- set 6 lines/inch.

7 -- set 7/72" line spacing.

9 -- set 8 lines/inch.

Run PSETMX without an argument to see a list of them.

PSETMX.ASM is the source for PSETMX.

NOTE:   For  convenience,  you may want to rename the version  of

PSET  you select to PSET.COM.   Also note that the PSET  programs

can only take one argument at a time.  You can run them more than

once to set up more than one option.

SDUMP.COM  -- This is a screen dump utility for IDS  Prism  color

printers.  Included with the program are modified versions of the

Zenith  demonstration  programs that will dump their displays  to

the printer when the F10 key is pressed.

SDUMP.DOC -- Instructions for SDUMP.

SDUMP.ASM -- The assembly source code for SDUMP.

B.BAS,  D.BAS, F.BAS, L.BAS, P.BAS, S.BAS, T.BAS -- Demo programs

for  use with SDUMP.   These programs should replace programs  of

the  same name on the demo disk supplied with H/Z-100  computers,

after first checking the segment address (DEF SEG statement).

FINDSEG.COM  -- This program prints out the current value of  the

DS  register  in  hex.   Use  it to find the  segment  value  for

programs like SDUMP, above.

FINDSEG.ASM -- The assembly source code for FINDSEG.

Program Authors:

TERMZ100,  DTERM, MENU, SDUMP,     Robert A. Metz

PRISM, PRINTEK, COLOR              Whirlpool Corp.

2000 US 33 N.

Benton Harbor, MI 49022

MENU modified by P. Swayne

PSET25, PSETMX, DIR100, DISKID     P. Swayne, HUG

FINDSEG                            DISKID developed from a

program by Marvin Fichter

(HUG disk 885-1213)

*****************************************************************

* The  Heath  Users'  Group is provided by Heath Company  as  a *

* service  to  its  members for the purpose  of  fostering  the *

* exchange of ideas to enhance their usage of Heath  Equipment. *

* As  such,  little or no evaluation of the program(s) is  per- *

* formed.   The  prospective user is hereby put on notice  that *

* the  program  may  contain faults the consequences  of  which *

* Heath Company in general and the Heath Users' Group (HUG)  in *

* particular cannot be held responsible.   The prospective user *

* is, by virtue of obtaining and using these programs, assuming *

* full risk for all consequences.                               *

*****************************************************************

* These  (this) program(s) were developed and tested on  Heath/ *

* Zenith Z-DOS 1.25  and believed to be free of program errors. *

* The  Heath  Users' Group cannot predict that  the  program(s) *

* will  perform  on future versions of  Z-DOS.   However,  if a *

* version dependent program surfaces,  REMark will publish  all *

* known fixes for all products.   Individual attention to prob- *

* lems  of this nature is not possible with a group as large as *

* ours.                                                         *

*****************************************************************

#                                                        *

******************