Configuring MICROSOFT WINDOWS NT 3.1 Video Miniport Driver

for

All Trident Microsystem Display Card

. Install Trident miniport driver:

1. Bootup to DOS

Assume that Windows NT is installed in C:\WINNT

2. Go to the driver where this diskette is located, i.e., A: or B:

3. Type NTINSTL C:\WINNT, and press ENTER

The NTINSTL.BAT will do following work:

A. Rename C:\WINNT\SYSTEM32\DRIVERS\VGA.SYS  TO

C:\WINNT\SYSTEM32\DRIVERS\VGA.OLD

B. Copy A:TRIDENT.SYS TO C:\WINNT\SYSTEM32\DRIVERS\VGA.SYS

C. Copy A:TRIDENT.SYS TO C:\WINNT\SYSTEM32\DRIVERS\TRIDENT.SYS

D. Copy A:VIDEO.INF TO C:\WINNT\SYETEM32\VIDEO.INF

. Select Display Resolution and color:

1. Run WINDOWS NT Setup

2. Select the resolution and color you want

3. When Messages Box asks "Do you want to use the currently installed

driver(s) or install new one(s) ", choose "Current"

4. Exit setup