HPLASINT

TABLE OF CONTENTS

OVERVIEW                                              1

IBM PC/XT & COMPATIBLES                               1

IBM AT & COMPATIBLES                                  2

386 SYSTEMS                                           2

TANDY COMPUTERS                                       3

ROM ADDRESS                                           4

TWO HARD DISK CONTROLLERS                             5

HARDWARE                                              6

SOFTWARE                                              7

INTERLEAVING                                          9

PARTITIONING                                         10

DRIVE LISTINGS AND OFFSETS                           11

COMPATIBLE/INCOMPATIBLE DRIVES LIST                  12

JUMPERING INSTRUCTIONS AND BIOS TABLES               14

TROUBLESHOOTING                                      18

OVERVIEW

This manual is intended to assist technical departments and

personnel in the installation of the Perstor PS180-8XT and PS180-

8AT hard disk controllers. It covers hardware and software

compatibility, necessary modifications of the PS180-8 bit

Controllers for operation in certain systems, and general

technical support operations for the PS180-8 bit Controllers.

IBM PC/XT AND COMPATIBLES

The Perstor PS180-8XT  Controllers are easily installed in

most PC/XT and compatible computer systems. The current ROM BIOSes

on the PS180 include version XT9-1.05, supporting IRQ and DMA

requests and have four hard disk cylinder and head configurations

in each of the four available drive tables. If two hard disks are to

installed in the system, both must be listed in the same drive table.

If they are not in the same table, a custom BIOS is necessary.

See page 14 for further information on these BIOS configuration tables.

Version XT9-1.05 operates in the same manner as version 1.03, but

it will allow the installation of two PS180-8XT Controllers in the same

system.  In addition, it will allow the utilization of low-level

formatting programs in other software packages.  The drive tables of

this version have been changed to accommodate some popular hard disks

that version XT9-1.03 did not support.

For XT and compatible systems having 286 accelerator cards,

386 accelerator cards, NEC V-20, or NEC V-30 chips, and the AT&T 6300

computers, it is advisable to use the AT9-1.03 version  of the BIOS

with the PS180-8XT Controllers.  This will make operation more efficient

and increase data transfer rate.

The AT&T 6300 computer has a resident ROM addressed at C800:0 which

must be disabled to operate with the PS180-8XT Controller. In addition,

the AT&T motherboard has two sets of 8 position dip switches; dip switch

0 and dip switch 1.  Position 3 of dip switch 1 (located to the back of

the motherboard) must be in the OFF position for this computer to operate

correctly with the PS180-8XT Controller.

Expansion boxes used with IBM PC compatibles have their own power

supply and bus expansion slots, and they use interface cards between the

the main computer and the expansion box. Some of these systems can be

can be used with the XT9-1.03 or XT9-1.05 Bios versions on the PS180-8XT

card with satisfactory results.  Other systems such as these will be

unable to perform data transfers and hard disk formats. If these problems

are encountered, they can be remedied by using the XAT9-1.03 BIOS available

from  Perstor Systems, Inc.

IBM AT AND COMPATIBLES

The Perstor PS180-8AT Controllers are easily installed in most AT and

compatible computer systems having up to a 10 MHZ bus speed and up to a

20 MHZ processor speed. With these 286 systems, the C-MOS must be set in

the system setup configuration to show that no hard disks are attached

prior to installing the  PS180-8AT Controller.

The current ROM BIOSes on the PS180 include versions AT9-1.03 and

AT9-1.07. They support programmed I/O transfers and have four hard disk

cylinder and head configurations in each of the four available drive

tables.  If two hard disks are to be installed in the system, both must

be listed in the same drive table.  If they are not in the same table, a

custom BIOS is necessary.  See page 14 for further information on these

BIOS configuration tables.

If you have the AT9-1.03 BIOS, a motherboard BIOS by Award requires

that either two drives be attached to the PS180-8AT or that drive 1 be

jumpered radially for the PS180-8AT to work properly. Using a PS180-8AT

(1.03 BIOS) with a system having a motherboard BIOS by AMI or DTK will

result in clock problems upon boot-up.  The hour will be correct, but the

minutes will always reset to 00.

Version AT9-1.07 eliminates the problems with the Award, DTK, and AMI

BIOSes. In addition, the AT9-1.07 Bios operates in the same manner as

version AT9-1.03, but it will allow the installation of two PS180-8AT

Controllers in the same system. In addition, it will allow the utilization

of low-level formatting programs in other software packages.  The drive

tables of this version have been changed to accommodate some popular hard

disks that are not supported with version 1.03.

386 SYSTEMS

The Perstor PS180-8AT Controllers are easily installed in most 80386

computer systems. In the system setup configuration, be sure to set the

C-MOS to show that no hard disks are attached prior to installing the

PS180-8AT Controller.

The current ROM BIOSes on the PS180 support programmed I/O transfers.

They have four hard disk cylinder and head configurations in each of the

four available drive tables. If two hard disks are to be installed in the

system, both must be listed in the same drive table. If they are not in

the same table, a custom BIOS is necessary. See page 14 for further

information on these BIOS configuration tables.

A motherboard BIOS by Award requires that either two drives be attached

to the PS180-8AT or that drive 1 be jumpered radially for the PS180-8AT to

work properly. Using a PS180-8AT with a system having a motherboard BIOS by

AMI or DTK will result in clock problems upon boot-up.  The hour will be

correct, but the  minutes will always reset to 00.

Version AT9-1.07 eliminates the problems with the Award, DTK, and AMI

BIOSes. In addition, version AT9-1.07 operates in the same manner as version

AT9-1.03, but it will allow the installation of two PS180-8AT Controllers in

the same system. It also allows the utilization of low-level formatting

programs in other software packages. The drive tables of this version have

been changed to accommodate some popular hard disks that are not supported

with version 1.03.

TANDY COMPUTERS

XT COMPATIBLES

The Tandy 1000 and 1000SX require modifications prior to installation

of the PS180-8XT Controller. Unlike IBM XTs and compatibles, which use IRQ5

IRQ5 for the hard disk controller interrupt, the Tandy computers use IRQ2

for the hard disk  controller and IRQ5 as an optional bus interrupt.

The IRQ5 interrupt line must be disabled to use the PS180-8XT. The

Tandy 1000SX has a four position dip switch. Position 2 of dip switch S2,

if set in the OFF position, will disable IRQ5. The Tandy 1000 may or may

not have this dip switch. If not, you must bend pin 13 of the 8259

interrupt control chip so that it is out of the socket. Be careful that

you do not break or remove this pin from the chip, however.

In addition, modifications must be made to the PS180-8XT Controller.

You must first cut a trace on the component side of the board at location

JMP3 as shown in the following diagram.

Next, on the solder side of the board, you must carefully  solder a

short jumper wire connecting the top of the B4 finger of the interface to

JMP3 as shown in the diagram on the next page.

This completes the modifications of the PS180-8XT and the Tandy 1000

and 1000SX.

286 TANDY COMPUTERS

The Tandy 286 computers require no modifications prior to installation

of the PS180-8AT Controller. However, only the AT9-1.03 BIOS has been

tested with the Tandy 3000 and 4000 computer systems.  Newer BIOS revisions

have not been tested with these computers.

ROM ADDRESS

The factory set ROM BIOS address of the PS180 8 bit Controller is

located at C800:0. If a conflict is noted at this address, the BIOS can be

relocated by making the following modifications to the solder side of the

controller.  Please refer to the following diagram for locations.

First, pin set 1-2 of JMP4 must be shorted to enable the ROM BIOS.

Then, to change the address, traces between various pin sets must be cut

according to the chart on the following page. If the address change you

wish to make is not listed, please call the technical support department

at Perstor Systems, Inc.

ADDRESS CHANGE          PIN SET TRACE THAT MUST BE CUT

-----------------------------------------------------------------

CA00                    3-4

CC00                    5-6

CE00                    3-4 and 5-6

D800                    9-10

-----------------------------------------------------------------

TWO HARD DISK CONTROLLERS

It is possible to install two PS180-8 bit Controllers in XT or

AT systems. Necessary equipment includes the XT9-1.05 or AT9-1.07 BIOS

BIOS (depending on which system you have), Perstor low-level formatting

software revision 1.2.2 or newer, Ontrack's Disk Manager partitioning

software, and two PS180 8 bit Controllers.

The primary (boot) controller requires no modification and has a

port address of 320 hex. Two hard disk drives must be attached to this

controller.  The secondary (co-resident) controller must be modified to

utilize port address 324 hex. If you have a Rev. 1.3 controller, this

can be done by cutting the trace between pin set 1-2 of JMP5 on the

solder side of the  board. If you have a Rev. 3.1 controller, you need

only remove the jumper plug from JMP5. Refer to the following diagram for

locations.

Next, connect the drive(s) to the respective controllers and proceed

with the PS2FMT low-level format program.  Following this step, you must

use Disk Manager by Ontrack to partition and high-level format all disk

drives.

HARDWARE

The majority of hardware products utilized today create no adverse

effects on the operation of the PS180 8 bit Controllers. A list of

compatible drives is on page 12. Perstor Systems,Inc. has successfully

tested the disk drives on this list; however, due to changes in product

by drive manufacturers, Perstor cannot warrant compatibility in all cases.

At present there are a few hardware options that pose some problems

when used with the Perstor Controllers. The following are details and

possible remedies for the hardware difficulties we have encountered.

1. The port address of the PS180 is at 320 Hex. In 286 and 386 computers

during boot time with the C-MOS set to "no hard disk drives installed",

the system will step up through the memory map until it finds a device

(BIOS ROM) that will take over. The Perstor BIOS, addressed at C800:0,

will finish the boot sequence.

2. WD 1002 Fox Floppy controllers do not work with the PS180. Use a

different stand alone floppy controller or replace it with the WD 1003

dual floppy/hard disk card.  We recommend using a stand alone floppy disk

controller such as the Kouwe Model KW530.  It supports 360K, 720K, and

1.2MB floppies in 286 computers only.

3. The Everex tape backup board must be set to DMA 1 if used with the Teac

tape backup machine.

4. The IBM BIOS used on clone system boards will display the F1  message.

You must exchange this BIOS with one designed for your specific motherboard.

We suggest using a Phoenix BIOS.

5.  With a Copy II PC option board installed in an XT or compatible system,

the interrupt level must be changed  because a conflict exists with the

interrupt of the PS180  controller.

6. The IBM XT BIOS dated November 2, 1982 will display the F1 error message.

You must exchange this BIOS with a newer version of the IBM XT BIOS.

7. Syquest 12MB SQ312R removable cartridge drives must have the following

modifications made: the W3, W8, and W9 positions must be open for proper

operation with the PS180.

8. If installing two Seagate hard disks, leave both terminating resistors

intact. This will allow the Seagate drives to perform more efficiently.

9.  Micro Science HH1120 RLL drives dated August 1988 and later with a

100+ nanosecond pulse width require a modification to the Perstor controller

artwork revisions below 3.0 for proper operation. On the solder side of

the PS180-8XT/-8AT at location JMP7, cut the trace between pins 2 and 3.

Connect a short jumper wire between pin 2 of JMP7 and pin 11 of component

U16 (the AN26S02).

10. Some system board BIOSes (such as the DTK) that support 3.5" floppy

drives will be incompatible with the PS180. These problems are currently

addressed. The DTK BIOS with 3.5" floppy support is compatible with the

AT9-1.03 BIOS, but the clock will reset the minutes to 00 during the boot

phase.  The AT9-1.07 BIOS will correct the clock problem, but will not

support 3.5" floppy drives.

11. To attach the drive activity LED to the PS180-8AT rev 1.3 controller,

disconnect pin 9 of location U12 (cut trace on solder side). Then connect

pin 8 of location U22 to pin 9 of location U12.

SOFTWARE

The majority of software programs utilized today do not conflict with

the PS180 8 bit Controllers. The following is just a partial list of

software programs, networking packages, and operating systems which ARE

compatible with the PS180 8 bit  Controllers.

Double DOS           Wordstar              First Publisher

Concurrent DOS       Word Perfect          Crosstalk

Norton Utilities     Microsoft Word        10 Net

Mace Utilities       Deskview              PC Slave

Fastback             RBase                 Novell Netware 86

Xtree                Fastcopy              3COM

SNAP                 V-Opt                 Gateway

V-Cache              Armour Systems        ARCnet

Copy II PC           Lotus 1-2-3           Direct Access

PKXARC               Harvard Graphics      DBASE3 and DBASE3 Plus

At present, however, there are a few software packages that pose

some problems when used with the PS180 8 bit Controllers. The following

are details and possible remedies for software incompatibilities we have

encountered.

1. MS-DOS has a partition bug. You should use PC-DOS for partitioning and

then continue with MS-DOS 3.2.

2. MS-DOS 3.31 allows you to make the entire drive bootable. However it

enlarges the sector size, so the software that checks the file allocation

table will fail.

3. Some software programs that access the standard ports 1F0 through 1F7

directly, programs that check the C-MOS for drive types, and programs which

do not go through a BIOS call are not compatible with the PS180 8 bit

Controller. Generally, however, these software packages do have the ability

to work  if a software switch is used.

4. With PC-MOS you must specify in the CONFIG.SYS file the exact area of

memory available. (i.e. Free memory = C9000, D9000). You must make sure

that the statement does not interfere with any other addressed cards.

5. The Sperry IT (AT compatible) with the PS180-8AT Controller installed

will not format floppy diskettes with the Sperry software. We suggest PC

TOOLS or other software to format floppy diskettes.

6. The PS2WP program on the Perstor diskette will clear all partitions

from the drive. It is a good idea to use this on all drives before

low-level formatting (PS2FMT).

7. If you are using PC-DOS 3.3, we recommend that you scan the drive for

defects two times. After you have low-level formatted the drive and

performed the first scan, do the following:

a.  Insert the PERSTOR Diskette into Drive A.

b.  Enter " PS2FMT " and press return.

c.  Enter whether the drive is Unit 0 or Unit 1.

d.  Enter the interleave value.

e.  When asked if you wish to format the drive, answer "NO".

f.  When asked if you wish to scan the drive, answer "YES" to perform

the second scan.

g.  When the second scan is complete, partition and high level format

the drive.

8. Spinwrite software by Gibson Research does not work with the XT9-1.03

or AT9-1.03 BIOS. You must use the revision XT9-1.05 or AT9-1.07 BIOS for

this program to be compatible with the PS180 8 bit controller.

9.  If you are using the Amiga 2000 operating system, you will need to

install an Amiga 2088 Bridgeboard for the PS180 8 bit Controller to work.

The PS180 should be installed on the "IBM compatible" side of the

Bridgeboard. Follow the instructions in both the Perstor Owner's Manual

and the Amiga 2088 Bridgeboard manual.

10. Windows 286 and 386 versions 2.03 and 2.1 are compatible with the

Perstor Controllers. However, you must first copy the Windows software into

a separate subdirectory (such as WINDOWS) and then perform the setup program

from there. Do not attempt to build or run the setup program for the

execution subdirectory WIN386.

10. Additional programs NOT compatible with the PS180 8 bit controllers

include:

Unix Operating System           Novell Netware 286

Xenix Operating System          Concurrent DOS version 1

OS/2                            Pick Operating System

INTERLEAVING

When physically sequential sectors on the disk are to be read, each

sector reaches the read/write head before a read or write operation can

be set up. The disk must make a complete rotation to pick up the next

sector. For example, when an attempt is made to read 31 sectors on a

particular track, 31 rotations are required. This takes up a considerable

amount of time, but performance can be significantly improved by proper

interleaving.

The optimum interleave value depends on many factors, such as the bus

and processor speed of your computer, number of buffers, drive speed,

controller capability, etc. The simplest way to determine the most suitable

interleave value for any particular system is through experimentation.

The PS180 8 bit Controller accepts any interleave value of 1 through

30. Though we suggest that you experiment to see which value gives you the

best performance, the following are some recommended interleave values for

different systems.

PROCESSOR SPEED   COMPUTER SYSTEM   INTERLEAVE   BEST SPEED

4.77 MHZ          XT or compatible      4        225 KB/sec.

4.77 - 8 MHZ      Turbo XT              5        225 KB/sec.

6 MHZ             AT or compatible      4        300 KB/sec.

8 MHZ             AT or compatible      4        300 KB/sec.

10 MHZ            AT or compatible      3        300 KB/sec.

12 MHZ +          AT or compatible      3        300 KB/sec.

_________________________________________________________________

By using a software program such as CORE or SPINTEST, you can

determine the data transfer rate of your system with any particular

interleave value. Follow these steps:

1) Low-level format the hard disk drive with PS2FMT for a minute and a

half with interleave 3.

2) Reboot the system with the DOS diskette in Drive A.

3)Insert the CORE or SPINTEST program in Drive A and activate.

4)Document the data transfer rate which is reported.

5)Repeat steps 1 through 4 with interleave 4 and document the

results.

6)Repeat these steps again using other interleaves such as 2, 5,

6, and 7, documenting the results.

7)Choose the value with the highest data transfer rate, and this

time format the entire hard disk.

PARTITIONING

Partitioning is necessary for accessing hard disk capacity above

33 megabytes. There are many software programs that can be used to

partition hard disks. The most popular ones include Disk  Manager by

ONTRACK, Speedstor by STORAGE DIMENSIONS, V-Feature Deluxe by GOLDEN

BOW, PC-DOS 3.3, MS-DOS 3.30, COMPAQ DOS 3.31, and ITT-DOS 3.1 and above.

All have been successfully tested with the PS180 8 bit Controllers.

Compaq and ITT programs partition and high-level format an entire hard

disk as one  bootable volume.

Special command lines are necessary for the execution of some

partitioning programs. They are outlined as follows:

DISK MANAGER    :  A>dm/m

SPEEDSTOR       :  A>parted/notype

V-FEATURE DELUXE: A>vol_def

(specify the controller as non-standard)

If you are using the AT9-1.07 or XT9-1.05 BIOS, you may use the

low-level formatting program provided with the above software packages.

The special command lines necessary for this purpose are:

DISK MANAGER    :  A>dm/m/p

SPEEDSTOR       :  A>hardprep/notype  (for initialization)

:  A>parted/notype    (for partitioning)

V-FEATURE DELUXE: A>vol_def

(specify the controller as non-standard)

DRIVE LISTINGS AND OFFSETS

The ROM BIOS address of the PS180 8 bit Controller is C800:0.  Below

are the individual drive listings and offsets of the BIOS. This information

is necessary to make custom BIOSes in the event that the hard disk you are

using is not listed in the BIOS configuration tables. Consult the IBM XT

technical manual, or contact Perstor or a distributor for further help.

AT9-1.03 and XT9-1.03 BIOS

TABLE 1                         TABLE 3

1st listing   offset 44         1st listing   offset 104

2nd listing   offset 54         2nd listing   offset 114

3rd listing   offset 64         3rd listing   offset 124

4th listing   offset 74         4th listing   offset 134

TABLE 2                         TABLE 4

1st listing   offset 84         1st listing   offset C4

2nd listing   offset 94         2nd listing   offset D4

3rd listing   offset A4         3rd listing   offset E4

4th listing   offset B4         4th listing   offset F4

Checksum      offset FFF

AT9-1.07 and XT9-1.05

TABLE 1                         TABLE 3

1st listing   offset 48         1st listing   offset C8

2nd listing   offset 58         2nd listing   offset D8

3rd listing   offset 68         3rd listing   offset E8

4th listing   offset 78         4th listing   offset F8

TABLE 2                         TABLE 4

1st listing   offset 88         1st listing   offset 108

2nd listing   offset 98         2nd listing   offset 118

3rd listing   offset A8         3rd listing   offset 128

4th listing   offset B8         4th listing   offset 138

Checksum      offset 1FFF

_________________________________________________________________

Using a drive with 1024 cylinders and 8 heads as an example,

convert the cylinders and heads from decimal values to hexadecimal

values. These values should be entered into the 1st drive listing of

Table 1 for the AT9-1.03 or XT9-1.03 Perstor ROM BIOS. Example

conversion:  1024 cyls = 400 H     8 heads = 08 H.

1ST DRIVE LISTING      OFFSET 44

offsets  44    45       46       47    48       49    4A

enter:  00    04       08       00    04       00    04

cylinders     heads     cylinders      cylinders

The checksum must be altered after the drive change. The two least

significant digits must be set to 00.

PS180 COMPATIBLE DRIVES LIST

MANUFACTURER      MODEL     CYLS    HEADS   ORIGINAL    CAPACITY

CAPACITY     W/PS180

________________________________________________________________

MINISCRIBE        3425      615      4       21.4         39.0

3053     1024      5       44.6         81.0

3085     1170      7       62.4        113.7

3650      809      6       42.2         77.0

6032     1024      3       26.7         48.7

6053     1024      5       44.6         81.0

6079     1024      5       44.6         81.0

6085     1024      8       71.3        130.0

6128     1024      8       71.3        130.0

8425      615      4       21.4         39.0

8438      615      4       21.4         39.0

8425F     615      4       21.4         39.0

8438F     615      4       21.4         39.0

SEAGATE           ST225     615      4       21.4         39.0

ST238     615      4       21.4         39.0

ST125     615      4       21.4         39.0

ST251     820      6       42.8         78.0

ST277     820      6       42.8         78.0

ST4096   1024      9       80.2        146.0

ST4051    977      5       42.5         77.0

NEWBURY DATA      NDR1065   918      7       55.9        101.0

NDR1085  1024      8       71.3        130.0

1140     1024      15     133.6        243.7

2190     1224      15     159.8        243.7

MAXTOR            1065      918      7       55.9        101.0

1085     1024      8       71.3        130.0

1140     1024      15     133.6        243.7

2190     1224      15     159.8        243.7

CDC WREN 2        94155-19  697      3       18.2         33.1

36        697      5       30.3         55.3

38        733      5       31.9         58.1

48        925      5       40.2         73.4

51        989      5       43.0         78.4

57        925      6       48.3         88.1

67        925      7       56.3        102.7

77        925      8       64.4        117.4

86        925      9       72.5        132.1

NEC               D5126     615      4       21.4         39.0

D5127     615      4       21.4         39.0

D5146H    615      8       42.8         78.1

D5147H    615      8       42.8         78.1

LAPINE            TITAN 20  615      4       21.4         39.0

MANUFACTURER      MODEL     CYLS    HEADS   ORIGINAL    CAPACITY

CAPACITY     W/PS180

________________________________________________________________

MICROSCIENCE      HH-1025   615      4       21.4         39.0

HH-1050  1024      5       44.6         81.0

HH-1120  1314      7       80.0        113.7

RODIME            3055      872      6       45.0         83.0

352       306      4       10.0         19.4

MICROPOLIS        1335     1024      8       71.3        130.0

PTI               PT225     615      4       21.4         39.0

PT338     615      6       32.1         58.6

PT238R    615      4       21.4         39.0

PT357R    615      6       32.1         58.6

PRIAM             ID45H    1024      5       44.6         81.0

ID130    1224      15     159.8        243.7

TOSHIBA           MK134     733      7       44.6         81.4

MK56FB    830     10       72.2        131.7

Note:  The PS180-8XT/-8AT controller supports only 1024 cylinders per

drive. The PS180-8 controller will increase capacity by less than 90%

on drives having more than 1024 cylinders.

DRIVES NOT RECOMMENDED FOR USE WITH

PERSTOR 200 SERIES CONTROLLERS

- CMI brand drives

- Vertex brand drives

- Fujitsu brand drives

- Quantum brand drives

- Tandon brand drives

- Shugart brand drives

- Any drive not listed on the compatible drives list

- Any drive having a window margin of less than + 30ns out of a

100ns window when measured on a Flexstar or Wilson tester.

JUMPERING INSTRUCTIONS AND BIOS TABLES

Follow these steps to configure your PS180 8 bit Controller for use

with your hard disk drive(s):

1) Locate Jumper block JMP6 on the PS180 Controller. Refer to the

diagram below.

2) Refer to the jumper tables on the following pages, making sure that

you match the table to the correct BIOS you have on your PS180 8 bit

Controller.

3) For the first drive, or if you are installing only one drive, use

the tables marked "Drive Unit 0". For a second drive, use the tables

marked "Drive Unit 1".

4) Locate the correct jumpering instructions for your disk drive by

matching the number of heads and cylinders in the table to your drive

specifications. If your disk drive configuration is not listed, you

will need a custom BIOS from your dealer.

5) Place the jumpers on the pin sets as listed in the table for your

disk drive.

6) If you are connecting a second drive (Unit 1), you must use the same

table number used for Drive Unit 0. If the drives you are connecting

are listed on separate tables, you will need a custom BIOS from your

dealer.

LISTING FOR AT9-1.03 OR XT9-1.03 BIOS

TABLE 1 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

306/4               OFF     OFF      OFF       OFF

640/4               ON      OFF      OFF       OFF

640/6               OFF     ON       OFF       OFF

1024/3              ON      ON       OFF       OFF

-----------------------------------------------------------------

TABLE 1 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

306/4               OFF     OFF      OFF       OFF

640/4               ON      OFF      OFF       OFF

640/6               OFF     ON       OFF       OFF

1024/3              ON      ON       OFF       OFF

-----------------------------------------------------------------

TABLE 2 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

615/4               OFF     OFF      ON        OFF

1024/5              ON      OFF      ON        OFF

1024/8              OFF     ON       ON        OFF

1024/15             ON      ON       ON        OFF

-----------------------------------------------------------------

TABLE 2 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

615/4               OFF     OFF      ON        OFF

1024/5              ON      OFF      ON        OFF

1024/8              OFF     ON       ON        OFF

1024/15             ON      ON       ON        OFF

-----------------------------------------------------------------

TABLE 3 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

697/5               OFF     OFF      OFF       ON

872/6               ON      OFF      OFF       ON

925/5               OFF     ON       OFF       ON

918/15              ON      ON       OFF       ON

-----------------------------------------------------------------

TABLE 3 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

697/5               OFF     OFF      OFF       ON

872/6               ON      OFF      OFF       ON

925/5               OFF     ON       OFF       ON

918/15              ON      ON       OFF       ON

-----------------------------------------------------------------

TABLE 4 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

615/4               OFF     OFF      ON        ON

733/5               ON      OFF      ON        ON

820/6               OFF     ON       ON        ON

1024/9              ON      ON       ON        ON

-----------------------------------------------------------------

TABLE 4 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

615/4               OFF     OFF      ON        ON

733/5               ON      OFF      ON        ON

820/6               OFF     ON       ON        ON

1024/9              ON      ON       ON        ON

-----------------------------------------------------------------

LISTING FOR AT9-1.07 OR XT9-1.05 BIOS

TABLE 1 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

615/6               OFF     OFF      OFF       OFF

615/8               ON      OFF      OFF       OFF

733/5               OFF     ON       OFF       OFF

830/10              ON      ON       OFF       OFF

-----------------------------------------------------------------

TABLE 1 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

615/6               OFF     OFF      OFF       OFF

615/8               ON      OFF      OFF       OFF

733/5               OFF     ON       OFF       OFF

830/10              ON      ON       OFF       OFF

-----------------------------------------------------------------

TABLE 2 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

612/4               OFF     OFF      ON        OFF

1024/5              ON      OFF      ON        OFF

1024/8              OFF     ON       ON        OFF

1024/15             ON      ON       ON        OFF

-----------------------------------------------------------------

TABLE 2 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

612/4               OFF     OFF      ON        OFF

1024/5              ON      OFF      ON        OFF

1024/8              OFF     ON       ON        OFF

1024/15             ON      ON       ON        OFF

-----------------------------------------------------------------

TABLE 3 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

615/4               OFF     OFF      OFF       ON

977/5               ON      OFF      OFF       ON

820/6               OFF     ON       OFF       ON

1024/9              ON      ON       OFF       ON

-----------------------------------------------------------------

TABLE 3 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

615/4               OFF     OFF      OFF       ON

977/5               ON      OFF      OFF       ON

820/6               OFF     ON       OFF       ON

1024/9              ON      ON       OFF       ON

-----------------------------------------------------------------

TABLE 4 FOR DRIVE UNIT 0      PIN SETS

CYLS/HEADS          5-6     7-8     13-14     15-16

925/5               OFF     OFF      ON        ON

925/9               ON      OFF      ON        ON

981/5               OFF     ON       ON        ON

918/15              ON      ON       ON        ON

-----------------------------------------------------------------

TABLE 4 FOR DRIVE UNIT 1      PIN SETS

CYLS/HEADS          1-2     3-4     13-14     15-16

925/5               OFF     OFF      ON        ON

925/9               ON      OFF      ON        ON

981/5               OFF     ON       ON        ON

918/15              ON      ON       ON        ON

-----------------------------------------------------------------

TROUBLESHOOTING

Proper Cable Attachment and Drive Select Jumpering

Refer to the diagram for J1 cable configuration.

J1 - 34 pin        2 drives attached with flat cable:

control cable        Jumper Drive Unit 0 as DS0

Jumper Drive Unit 1 as DS1

2 drives attached with twist at Drive Unit 0:

Jumper both drives as DS1

2 drives attached with twist at Drive Unit 1:

Jumper both drives as DS0

J2- 20 pin        Attach to hard drive data interface ofDrive

data cable           Unit 0

J3 - 20 pin        Attach to hard drive data interface of Drive

data cable           Unit 1

Pin 1 of all three cables (normally a red stripe) should be

toward the mounting bracket of the Perstor Controller. In some cases,

20 pin data cables will work properly with a standard hard disk

controller, but when attached to the Perstor Controller these cables

could fail.

To test the data cable, low-level format (PS2FMT) for approximately

1 minute and reboot the computer. Enter PS2FMT again and watch for a

warning message. If this message appears stating that the drive has

already been formatted, the data cable is operating properly. If no

warning message appears, you should replace the data cable.

Other Troubleshooting Tips

1)  If the controller passes the test but the drive does not  work,

check to make sure that the cables are connected properly.

2)  If the controller passes the test and only one drive works, check

to make sure that the cables are connected to the correct drives

and that the red stripe of the cable is aligned properly.

3)  If the screen displays: "Invalid Drive Specification" check to

make sure that the cables are attached properly and that the drive

has been partitioned correctly and activated.

4)  If the screen displays: "Error Reading Drive C (or D)" check to

make sure the cables are attached to the correct drive(s), check

for cable damage, and low-level format the drive(s) again.

5)  If the screen displays a 1701 error message and the controller is

not able to recognize the drive(s), reboot the system. If the drive

is still not recognized, check to make sure the cables are attached

properly and that the power plug is connected to the drive(s).

6)  If the disk drive has a solid red light, check for proper alignment

of the red stripe on J1 cable, check your power supply, and check

the power line voltage.

7)  The low-level format (PS2FMT) should take approximately 10 minutes

per 20 megabytes of the original capacity. If this process takes

longer, test data cables as outlined above.  Replace if necessary.

This test was performed on a XT at 4.77 MHZ. On faster systems this

time will vary.

8)  If the drive does not boot, check to see if the first partition

is activated.

9)  Always check to make sure that jumper block JMP6 is jumpered for

the correct disk drive(s) and BIOS.

10) If an AT compatible freezes up during the initial startup after

installing the PS180 8 bit Controller, check the C-MOS to see if

there are any hard disks installed. If so, you must specify that

no hard disks are attached to the original controller before

re-inserting the PS180 8 bit Controller into the computer.

11) If the Perstor copyright message does not appear at the top of the

screen upon boot-up, check the following:

a) there should be power to the drive(s)

b) the Perstor BIOS should be enabled

c) the C-MOS should have no drives installed

d) the J1 cable should be attached correctly

e) the Award BIOS should have the AT9-1.07 or XT9-1.05 BIOS on the

PS180-8AT or PS180-8XT

Debug Routines

Copyright Message            - D C800:0

Install ROM BIOS             - A100                    (return)

Address call C800:3      (return)

Address ret              (return)

- G=100                   (return)

Number of Drives Recognized  - d 40:75

First Line  __ __ __ __ __  02 05

(two drives)

To Change Number of Drives   - e 40:75

Address 00.0X   (change X to #)

Reset Controller & Drive     - O 321 01

Status                       - i 321    should return C0