%OP%DESystem Calls Reference

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,5,74%OS_MOP, open memory (allocate memory pool)

RST 20H, DEFB $4E

IN:

OUT, if call successful:

OUT, if call failed:

Registers changed after return:

Notes:

Bugs:

Related calls:

OS_BIX, bind in extended address

OS_BOX, restore bindings after OS_BIX

OS_FC, select fast code (fast bank switching)

OS_MAL, allocate memory

OS_MCL, close memory (free memory pool)

OS_MFR, free (previously allocated block) memory

OS_MGB, get current bank binding in segment

OS_MPB, set new bank binding in segment

%P0%

%CO:B,5,69%

A = memory mask arguments (several arguments OR'ed together):

BC = 0, always

Fc = 0

IX = memory pool handle

Fc = 1

A = error code:

A.FBCDEHL/..IY same

.F......./IX.. different

MM_EXC and MC_CI use the same argument bit. We don't know why, but

believe its the same function (Internal & Explicit slot usage).

If only the memory segment mask are specified, it is probably

default'ed to MM_MUL - for the rest of the arguments, we don't know.

MM_FIX is probably an internal usage.

%CO:C,5,64%

%H4%Destination segment (allocated memory addressed for segment):%H4%

MM_S0 ($00), segment 0

MM_S1 ($40), segment 1

MM_S2 ($80), segment 2

MM_S3 ($C0), segment 3

%H4%Source of memory (to be allocated):%H4%

MM_FIX ($02), fixed workspace (not subject to swapping)

%H4%or%H4%

MM_SLT ($04), explicit slot usage, mask also following:

%H4%Allocation variations%H4%:

MM_EXC ($10), exclusive use of bank

MM_MUL ($20), use multiple banks

RC_NA ($06), handle not available

RC_ROOM ($07), no room

%CO:D,5,59%

MC_CI ($10), internal memory

MC_C1 ($01), card 1

MC_C2 ($02), card 2

MC_C3 ($03), card 3

%CO:E,5,54%%CO:F,5,49%